| 6 | Equations Segment In Depth |
| --- | --- |

Contents

µPairing	193

Implicit Pairing Rules and Behavior	193

Output Pairing	195

Input Pairing	196

MACH 1xx Designs	196

MACH 2xx (Except MACH215) Designs	198

MACH 4xx and MACH215 Designs	199

Explicit Pairing Rules and Behavior	199

Copying Logic with Braces { }	200

Output Pairing	200

Input Pairing	203

Polarity	205

The Two Components of Polarity	205

Controlling Polarity from the Equation	205

Controlling Polarity from the Pin Statement	206

Controlling Polarity from CASE Statements	206

Functional Equations	209

Controlling Three-State Output Buffers	209

Controlling Clocks	210

Specifying a Rising-Edge Clock	211

Specifying a Falling-Edge Clock	211

Specifying a Product-Term Clock	212

Global Clock Acquisition	212

Controlling Set and Reset	214

Sharing Set and Reset	215

Vectors	216

Ranges of Pins or Nodes	216

Comma-Delimited Vectors	218

Radix Operators	219

IF-THEN-ELSE Statements	221

CASE Statements	222

Building State Machines with CASE Statements	224

Multiple State Machines	230

The "Don't-Care" Logic-Synthesis Option	237

MINIMIZE_ON and MINIMIZE_OFF	241

# PairingXE "pairing"§

The Fitter supports the MACH device's ability to pair input registers and output registers with I/O pins to provide registered or latched I/O.  Pairing is supported in three broad categories:

r	Implicit Pairing XE "implicit pairing "§XE "pairing:implicit"§occurs automatically when an output pin is declared as REGISTERED or LATCHED but not explicitly paired in the design file with a macrocell.  (There is no implicit pairing for inputs.)

r	Automatic Pairing XE "automatic pairing"§XE "pairing:automatic"§occurs when all four of the following conditions are met: 1) an input or output pin is declared and defined, 2) a node is declared and defined, 3) the pin and node are used in such a way as to imply a paired relationship, and 4) no PAIR keyword is present in the appropriate PIN or NODE statement.  (In MACH 4xx designs, there is a fifth condition: the Use automatic pin/node input pairing option of the Logic Synthesis Options form must be set to "Y.")

r	Explicit Pairing XE "explicit pairing "§XE "pairing:explicit"§occurs when a PIN statement contains the keyword PAIR and references the logical name of an input node (input pairing) or when a NODE statement contains the keyword PAIR and references the logical name of an I/O pin (output pairing).  Explicit pairing is the best way to ensure that you get the behavior you expect from your design.

## Implicit Pairing Rules and BehaviorXE "implicit pairing rules and behavior"§XE "pairing:implicit, rules"§

Most output equations can be implemented using implicit pairing.  The exception is the case in which you want to be able to use both pin and node feedback from the same pin; in such cases, you must use explicit pairing.

Implicit output pairing occurs when the existence of a register or latch is specified in a PIN statement that does not contain the PAIR keyword.  Implied output pairing can take either of two forms:

r	Declaring a pin signal as REGISTERED or LATCHED

r	Referring to a pin's feedback signal as REGISTERED or LATCHED

Implicit pairing emulates PAL22V10 and MACH1xx/2xx device behavior, as shown in the following design example below and diagram.

...

PIN	?	IN1

PIN	?	IN2

PIN	?	IN3

PIN	?	OUT1	REGISTERED

PIN	?	OUT2	COMBINATORIAL
PIN	?	CLOCK

...

EQUATIONS

OUT1	= IN1 * /IN2
OUT1.CLKF	= CLOCK

OUT2	= OUT1 * IN3

...

µ §

When you use the implicitly-paired pin's name on the right side of an equation, feedback is always routed from the register's output (as it would be in a PAL22V10 design).

The term "automatic pairing" describes the case in which the compilation program generates a missing PAIR keyword in a PIN or NODE statement to enable pairing behavior that is implied by a Boolean equation.  Automatic pairing occurs only if both the pin and node are declared but not explicitly paired (refer to "Implicit Pairing," above, for information on how the Fitter creates a node that was not declared but was implied in the design file).

For MACH 4xx devices, automatic input pairing (not available for MACH 1xx/2xx/3xx devices) can occur only when the Use automatic pin/node input pairing? option of the Logic Synthesis Options form is set to "Y."  Automatic output pairing is available for MACH 3xx/4xx devices at all times.  For MACH 1xx/2xx devices, automatic output pairing is available only when the Use automatic pin/node pairing? option of the Logic Synthesis Options form is set to "Y."

Unlike implicit pairing, which creates an output node for you when you have declared only the output pin, automatic pairing occurs only when you have declared both the pin and the node to be paired.

### Output Pairing

For MACH 3xx/4xx devices, automatic output pairing is always enabled.  For MACH 1xx/2xx devices, automatic output pairing occurs only when the Use automatic pin/node pairing? option is set to "Y."  Automatic output pairing occurs when you declare a pin and a node and use the identical Boolean expression on the right side of the equations for both signals, as in the following example:

PIN	?	OUT2	REGISTERED
NODE	?	BR2	REGISTERED
...
EQUATIONS
OUT2	= A * B * /(C + D)
BR2	= A * B * /(C + D)

If you compile the example above, regardless of the Use automatic pin/node input pairing? option setting, the software completes the pairing for you by modifying the NODE statement to look like this:

NODE  ?   BR2        REGISTERED    PAIR    OUT2

### Input Pairing

Automatic input pairing occurs when you declare a pin and a node and use the pin signal name, by itself, on the right side of the equation for the node, as shown below:

PIN	?	P2	REGISTERED
NODE	?	N2	REGISTERED
...
EQUATIONS
N2 = P2

#### MACH 1xx DesignsXE "automatic input pairing:MACH 1xx designs"§

MACH 1xx devices have neither input registers nor buried registers that can be paired directly with input pins.  Instead, registered input equations are implemented as sum-of-products logic at an output register (that is, as feedback from a standard output macrocell), which therefore becomes unavailable for other purposes.

Equations that would be interpreted as input-pairing equations in other devices are implemented, in MACH 1xx devices, as output equations.  This includes explicit input-pairing equations, which are automatically rewritten by the MACHXL application as output equations, as shown in the following example.

Example 1

...

CHIP   EMULATED_INPUT_REG   MACH111

...

PIN   ?   IN1   IPAIR  INODE

PIN   ?   IN2

PIN   ?   CLOCK

NODE   ?  INODE  REGISTERED

PIN   ?   OUT2   COMBINATORIAL

EQUATIONS

INODE      = IN1

INODE.CLKF = CLOCK

OUT2       = IN2 * INODE

The following diagram shows how the design fragment presented on the previous page would be implemented on a device that has dedicated input registers.

µ §

However, because the MACH111 device (like all MACH 1xx devies) does not have input registers, the MACHXL software automatically rewrites registered-input equations in such a way that the input signals are registered using a general-purpose macrocell.  In this example, the buried macrocell ONODE registers the input IN1 using a standard output equation (ONODE = IN1) and the feedback from ONODE is used by OUT2 as if it were the output from a dedicated input register.  The following design fragment and the figure on the next page illustrate how this is done.

Example 2

...

CHIP   EMULATED_INPUT_REG   MACH111

...

PIN   ?   IN1

PIN   ?   IN2

PIN   ?   CLOCK

NODE  ?   ONODE  REGISTERED

PIN   ?   OUT2   COMBINATORIAL

EQUATIONS

ONODE      = IN1

ONODE.CLKF = CLOCK

OUT2       = IN2 * ONODE

µ §

The implementation of registered inputs as output equations means that the inputs are subject to the same propagation delay as signals fed back from any other output registers.

#### MACH 2xx (Except MACH215) DesignsXE "automatic input pairing:MACH 2xx (except MACH215) designs"§

Automatic input pairing occurs only if both of the following conditions are met:

1.	The node uses an implied clock or a single-literal clock that is preplaced at or placeable at a global clock pin.

2.	The node is defined as a D-type register or a latch.

If both conditions are met, the software completes the pairing by modifying the PIN statement in Example 2, above, to look like this:

PIN  ?  IN1        IPAIR    ONODE

If only condition 1 is met, the equation is implemented by passing the pin signal through the central switch matrix and the sum-of-products array to a buried macrocell (not a dedicated input register).  This uses more interconnect resources than a dedicated input register and introduces a propagation delay into the signal path.

If you want the input signal to pass through central switch matrix instead of being routed directly to the macrocell, set the Use automatic pin/node pairing? option to "N."

#### MACH 4xx and MACH215 DesignsXE "automatic input pairing:MACH 4xx and MACH215 designs"§

Automatic input pairing occurs only if all of the following four conditions are met:

1.	The Use automatic pin/node input pairing? option is set to "Y."

2.	There are no set or reset equations defined for the node (because the input macrocell has no set/reset capability).

3.	The node uses an implied clock or a single-literal clock that is preplaced at or placeable at a global clock pin.

4.	The node is defined as a D-type register or latch (not T-type).

If all four conditions are met, the software completes the pairing by modifying the PIN statement in Example 2, above, to look like this:

PIN  ?  P2        PAIR    N2

If condition 1 is met but one or more of the other conditions is not met, the equation is implemented by passing the pin signal through the central switch matrix and the sum-of-products array to a buried macrocell (not a dedicated input register).  Such an implementation allows you to make use of the buried macrocell's set and reset functions, but uses more interconnect resources than a dedicated input register and introduces a propagation delay into the signal path.

If you want the input signal to pass through central switch matrix, and to use a buried register rather than an input register, set the Use automatic pin/node input pairing? option to "N."

## Explicit Pairing Rules and BehaviorXE "explicit pairing rules and behavior"§XE "pairing:explicit, rules"§

Explicit pairing allows you direct control over feedback paths, and allows you to control registered/latched inputs precisely.  The following general rules apply to both input and output pairs that are explicitly defined:

r	If you write functional equations for one partner of the pair and not the other, the missing functional equations are automatically applied to both the pin and the node.

r	If you write different equations for a paired pin and node, the Fitter generates a warning message and disregards the PAIR statement.

### Copying Logic with Braces { }

XE "copying logic with braces { }"§XE "{ }, copying logic with"§XE "braces, copying logic with"§If you write an equation for a pin or node, you can copy the expression on the right side of the equation to another pin or node.  You do this by placing the name of the pin or node for which the full equation was written, surrounded by braces (the symbols { and } ) on the right side of an equation for the pin or node to which you want to copy the equation:

P2 = IN3 * /IN4 + IN5 * /IN6	;defines behavior for pin P2
N2 = {P2}	;copies "IN3 * /IN4 + IN5 * /IN6"
	;to node N2

### Output PairingXE "output pairing"§XE "pairing:output"§

Use output pairing to associate a node with an output.  To perform explicit output pairing, write the PIN statement exactly as you normally do:

PIN  ?  OUT2  REGISTERED

Then, write a NODE statement with the same storage type, add the keyword PAIR, and add the logical name of the output pin with which you want to pair the node:

NODE  ?  BR2  REGISTERED  PAIR  OUT2

In the EQUATIONS segment of the design file, write equations for either the pin or the node.  For example:

OUT2	= IN1 * IN2
OUT2.CLKF	= CLK2
OUT2.RSTF	= INIT

Finally, copy all the primary equation from one paired partner to the other using the { } symbols, as follows:

BR2	= {OUT2}

The pin and node are now paired and share identical logic.  It is good practice to copy all functional equations as well as the primary equation, as shown on the next page.

BR2.CLKF	= {OUT2.CLKF}
BR2.RSTF	= {OUT2.RSTF}

XE "copying equations"§Copying all equations explicitly makes your intent clear to someone reading your design.  (However, if you copy only the primary equation, the MACHXL software automatically copies all related equations for you.)

| ã | Note: Feedback routing XE "feedback routing "§differs between pins that are implicitly paired and pins that are explicitly or automatically paired. Explicitly and automatically paired pins do not emulate PAL22V10 behavior. To get node feedback, you now must use the node name, not the pin name, on the right side of an equation. Using the pin name on the right side of the equation specifies pin feedback, which may or may not correspond to node feedback, depending on the state of the pin's output buffer. |
| --- | --- |

In the following design example, the feedback signals from pin OUT2 are routed from the pin as well as from the node, to illustrate how to specify each type of behavior.

...
CHIP   NODE_FB   MACH435
...
PIN	?	IN1
PIN	?	IN2

PIN	?	IN3

PIN	?	IN4

PIN	?	RESET

PIN	?	PRESET
PIN	?	CLOCK
PIN	?	OUT1	REGISTERED 	;DEFINES PIN AS REGISTERED
PIN	?	OUT2	REGISTERED 	;DEFINES PIN AS REGISTERED

PIN	?	OUT3	COMBINATORIAL	;DEFINES PIN AS COMBINATORIAL

PIN	?	OUT4	COMBINATORIAL	;DEFINES PIN AS COMBINATORIAL
PIN	?	OUT5	COMBINATORIAL	;DEFINES PIN AS COMBINATORIAL

NODE	?	BR2	REGISTERED PAIR OUT2 ;DEFINES OUTPUT PAIRING

Continued...

...Continued

EQUATIONS
;----- ILLUSTRATES IMPLIED BEHAVIOR (DEFAULT) ------------------
OUT1      = IN1        ;SINCE NO NODE IS SPECIFIED, THE MACHXL

;SOFTWARE ASSIGNS ONE (BECAUSE PIN WAS

;DEFINES AS REGISTERED)

OUT3      = IN2 * OUT1 ;BY DEFAULT, FEEDBACK FROM A PIN PAIRED

;WITH A NODE BY IMPLICATION (AS WAS PIN

;OUT1) IS ROUTED FROM THE BURIED NODE
OUT1.CLKF = CLOCK

;----- ILLUSTRATES EXPLICITLY-SPECIFIED BEHAVIOR ---------------

OUT2      = IN3 * OUT1

BR2       = {OUT2}	 ;USING {} IS THE BEST WAY TO ENSURE

;THAT THE PIN AND NODE USE IDENTICAL

;EQUATIONS.

BR2.CLKF  = CLOCK

OUT4      = IN4 * BR2	 ;YOU MUST USE THE NODE NAME TO GET

;NODE FEEDBACK

OUT5      = IN5 * OUT2 ;IF YOU USE THE PIN NAME, YOU GET FEEDBACK

;FROM THE PIN

The next two figures illustrate the default behavior of pin OUT1 as well as the explicitly-specified behavior of pin OUT2.

µ §

µ §

### Input PairingXE "input pairing"§XE "pairing:input"§

Use input pairing to define a registered/latched input.  To perform explicit input pairing, write the NODE statement exactly as you normally do:

NODE  ?  IR2  REGISTERED

Then, write a PIN statement without any storage type, add the keyword PAIR, and add the logical name of the node with which you want to pair the pin:

PIN  ?  REG_IN2  PAIR  IR2

In the EQUATIONS segment of the design file, write an equation for the NODE ONLY, using the pin name, by itself, on the right side of the equation.

IR2 = REG_IN2

r	Use the node name on the right side of an equation to get registered input from the pin:

OUT5 = IR2 * IN4 ;IR2 is registered input

r	Use the pin name on the right side of an equation to get combinatorial input from the pin:

OUT7 = REG_IN2 * IN5 ;REG_IN2 is combinatorial input

The following figure shows how input from pin REG_IN2 is routed to outputs OUT5 and OUT7.

µ §

# PolarityXE "polarity"§

An output pin that goes high when the corresponding equation is true is called active highXE "active high"§.  An output pin that goes low when the corresponding equation is true is called active lowXE "active low"§.

MACH devices have outputs that can be configured as either active-high or active-low output.  This valuable feature is known as programmable polarity.

## The Two Components of PolarityXE "polarity:components of"§

In a MACHXL design file,  the active high/low nature of each pin is a function of its polarity definition.  Polarity is defined in both the pin statement and the output equation.

The polarity rule for MACHXL design files is defined below.

r	If the equation and pin statement have the same polarity, the output is active high.

r	If the equation and pin statement have opposite polarity, the output is active low.

Active-low polarity in a MACHXL design file is indicated with a slash, /.

## Controlling Polarity from the EquationXE "polarity:controlling from equations"§

Always use active-high (non-complemented) pin names in the pin statements.

PIN  14  O1	COMBINATORIAL	;output
PIN  15  O2	REGISTERED	;output

With active-high pin statements, the polarity of the output is the same as the polarity of the equation.  Create an active-low equation by placing a slash before the pin name on the left side of the equation.

O1 = I1 * I2		;active-high output
/O2 = I1 * I2		;active-low output

Advantage of this Method:

Can be used to control the polarity of pins and nodes.

| ã | Note: When the 22V10/MACH1XX/2XX S/R Compatibility? option (not available for MACH 1xx/2xx devices) is set to "N," equations are minimized fully, the polarity of each equation is adjusted using the macrocell's polarity XOR. When the 22V10/MACH1XX/2XX S/R Compatibility? option is set to "Y," the only way to ensure optimal minimization is to set the Ensure polarity after minimization option to "Best for device" (in which case polarity may differ from that specified in the design file). |
| --- | --- |

## Controlling Polarity from the Pin StatementXE "polarity:controlling from PIN statement"§

This method of controlling polarity applies to pins only.

Always use the non-complemented pin name on the left side of output equations, as shown below.

OUT1 = I1 *  I2  ;These equations have the polarity 
 OUT2 = I1 *  I3  ;of pins OUT1 and OUT2, respectively.

/OUT3 = I1 * /I5  ;<-Do not complement the left side like this!

Using this method, the polarity of the output is always the same as the polarity of the pin or node statement.

PIN  14   OUT1  COMBINATORIAL	;active-high output

PIN  15  /OUT2  REGISTERED	;active-low output

Advantage of This Method:

It is easy to tell from the pin names in the DECLARATION segment which pins and nodes are active-high and which are active-low.

## Controlling Polarity from CASE StatementsXE "polarity:controlling from CASE statements"§

When the MACHXL software processes a CASE statement, the CASE statement is converted to as many Boolean equations as are required to express the logic described in the CASE statement.  The polarity of each output signal is determined like that of any other output signal: by the interaction between two elements:

r	The polarity or the signal's declaration

r	The polarity of the first equation encountered for that signal.

Suppose that the pins OUTPUTS[1..3] are declared as follows:

PIN  ?  OUTPUTS[1..3]

Suppose also that the first equation for these pins is in the following CASE statement:

...

CASE  (STATEBIT[1..2])

BEGIN

INPUT1: BEGIN

IF (CONDITION1) THEN

BEGIN

OUTPUTS[1..3] = #B010

END

ELSE

BEGIN

OUTPUTS[1..3] = #B001

END

END

...

END  ; END OF CASE CONSTRUCT

...

The first statement that references the vector OUTPUTS[1..3] is:

OUTPUTS[1..3] = #B010

This statement is expanded into the following three Boolean equations:

/OUTPUT[1] = VCC
 OUTPUT[2] = VCC
/OUTPUT[3] = VCC

The pins OUTPUT[1] and OUTPUT[3] are configured as active-low pins because their PIN declarations are uncomplemented while the first equation processed for each pin is complemented.

If you want these pins to be configured as active-high pins, you must ensure that the first equation for each pin that the compiler encounters is uncomplemented.  You can do this two ways:

By inserting dummy equations for each signal before the CASE statement.
The dummy equation OUTPUT[1] = IN1 * /IN1 is ideal in that it provides an uncomplemented equation while at the same time leaving the design's functionality unchanged, insofar as the condition IN1 * /IN1 can never occur.

By inserting a dummy IF..THEN..ELSE statement at the beginning of the CASE statement, as shown in the following example:

...

CASE  (STATEBIT[1..2])

BEGIN

INPUT1: BEGIN

;----------------begin dummy statement----------
           IF (IN1 * /IN1) THEN

BEGIN

OUTPUTS[1..3] = #b111

END

;----------------end dummy statement------------
           IF (CONDITION1) THEN

BEGIN

OUTPUTS[1..3] = #B010

END

ELSE

BEGIN

OUTPUTS[1..3] = #B001

END

END

...

END  ; END OF CASE CONSTRUCT

...

# Functional EquationsXE "functional equations"§XE "equations:functional"§

Functional equations control output buffers, clocks, preset, and reset.  All functional equations take the form 

Pin_or_node_name.Function = Boolean_expression

Valid substitutions for Function are:

r	.TRST

r	.CLKF

r	.SETF

r	.RSTF

Complemented equations are illegal for all functional equations except .CLKF functional equations.

## Controlling Three-State Output BuffersXE "three-state output buffers, controlling"§XE ".TRST"§XE "output buffers"§

The MACH device macrocell can be configured to use one of its product terms to control output enable.  If you write an output equation for a pin but do not write an output-enable equation for it, its output is unconditionally enabled.

To control the three-state buffer yourself, use a .TRST functional equation with the following syntax.

Pin_name.TRST = Product_term

You have three options when defining a .TRST equation.

r	Enable the output buffer at all times.

r	Disable the output buffer at all times.

r	Enable the output buffer under certain conditions.

To enable the output buffer at all times, set the .TRST equation equal to VCC.  To disable the output buffer at all times, set the .TRST equation equal to GND.  The following example unconditionally enables pin A and disables pin B.

A.TRST = VCC	;enables output A unconditionally
B.TRST = GND	;disables output B unconditionally

To enable the output buffer under certain conditions, set the .TRST equation equal to a Boolean expression.  The following example enables pin B when the signal GO is high and STOP is low.  GO and STOP must be defined as pins or nodes in the declaration segment of the PDS file.

B.TRST = GO * /STOP

## Controlling ClocksXE "clocks, controlling"§XE ".CLKF"§

Use a .CLKF functional equation to control the clock signal to flip-flops in a MACH device.

To control the clock of a flip-flop, you define the clock signal with a pin statement in the declaration segment of the PDS file.  Then you use this signal in a .CLKF functional equation.

Pin 20 is a clock pin on the MACH435 device.  The following example assigns the name CLK to this pin using a pin statement.

;----------------Declaration Segment----------------------
PIN       20	CLK
PIN       ?	A        		;INPUT
PIN       ?	B        		;INPUT
PIN       ?	AREG	REGISTERED	;OUTPUT
;----------------Equations Segment------------------------
AREG  = A + B
AREG.CLKF = CLK

Place the PIN statement in the declaration segment of the PDS file.  You can assign any name that is valid for pins and nodes to the clock pin (it helps to use a name that is readily associated with the clock signal).  Then you use the clock signal in a .CLKF functional equation in the equations segment of the PDS file to control the clock of the register associated with output pin AREG.

| ã | Note: Each MACH device has a default clock pin that the MACHXL software uses to clock any register for which you do not specify a clock signal. In general, it is best to specify clock signals for all registers explicitly. Refer to Chapter 10, "Device Reference," for details on the default clock pin of your target MACH device. |
| --- | --- |

### Specifying a Rising-Edge ClockXE "rising-edge clock"§XE "clock:rising-edge"§

To specify a rising-edge clock, do the following:

r	Write the PIN statement for the clock pin using an uncomplemented pin name.

r	Write an active-high (not complemented) .CLKF equation.

Example

PIN  20  CLK
PIN  ?   OUT2    REG
...
EQUATIONS
OUT2.CLKF = CLK
...

### Specifying a Falling-Edge ClockXE "falling-edge clock"§XE "clock:falling-edge"§

To specify a falling-edge clock write an active-low clock equation.  There are two recommended ways to do this:

Option A (MACH 3xx/4xx and MACH215 designs only)
Do the following:

r	Write the PIN statement for the clock pin using an uncomplemented pin name.

r	Complement the left side of the .CLKF equation.

Example

PIN  20  CLK
PIN  ?   OUT2    REG
...
EQUATIONS
...
/OUT2.CLKF = CLK

Option B
Do the following:

r	Write the PIN statement for the clock pin using a complemented pin name.

r	Write an active-high (not complemented) .CLKF equation.

Example

PIN  20 /CLK
PIN  ?   OUT2    REG
...
EQUATIONS
...
OUT2.CLKF = CLK

### Specifying a Product-Term ClockXE "product-term clock"§XE "clock:product-term"§

(MACH 3xx/4xx and MACH215 designs only)

Asynchronous macrocells can use product-term clocks or clock-pin clocks.  To specify a product-term clock, place a Boolean expression on the right side of the .CLKF equation.

Example

PIN  ?   IN2
PIN  ?   IN3
PIN  ?   OUT2    REG
...
EQUATIONS
...
OUT2.CLKF = IN2 * IN3

### Global ClockXE "global clock acquisition"§XE "clock:global, acquisition"§ Acquisition

Clock signals originating at the MACH device's global clock pins are normally routed differently from signals originating at input or I/O pins.  Under certain circumstances, however, you may want to allow the Fitter to route such signals as product-term clocks rather than through the block clock mechanism.

The Global clocks routable as Pterm  clocks? setting on the MACH Fitting Options form (set to "N" by default) allows global clock signals to be routed either as global clocks or as product-term clock signals when set to "Y."  Clocks routed through the central switch matrix as product terms are somewhat slower than block clocks, but users of such clocks sometimes have more partitioning flexibility.

| ã | Note: The MACH465 and MACH 1xx/2xx devices do not allow global clock signals to be routed through the switch matrix but only through the block clock mechanism. When setting fitting options for a design written for one of these devices, the Global clocks routable as Pterm clocks? option does not appear in the MACH Fitting Options form. |
| --- | --- |

A floating, non-grouped, single-literal clock may be classified by the software as a global clock under the following circumstances:

r	At least one of the global clock pins is not in use (that is, the number of forced global signals is less than the number of clock pins)

r	The clock signal under consideration is used as a clock more frequently than other clock signals that are eligible (but not forced) to be global clocks

| ã | Note: Some MACH435 and MACH465 designs are difficult to fit if all of the global clock pins are populated. To prevent the fitter from assigning to global clock pins signals that do not need to be global, set the Reduce Non-forced Global Clocks? option of the MACH Fitting Options form to "Y." (The Reduce Non-forced Global Clocks? option is not available for MACH 1xx/2xx devices.) |
| --- | --- |

However, any single-literal clock must be classified by the software as a global clock when one or more of the following conditions exists:

r	The clock under consideration is preplaced at a global clock pin.  (Use this fact to force single-literal clock signals to be global by preplacing them at one of the device's global clock pins.)

r	The clock under consideration is used to clock an input register or latch and there is a global clock available (otherwise, the input pairing is discarded and a warning issued).

r	The clock under consideration is used to clock a macrocell for which both non-GND .SETF and .RSTF equations are defined.

r	(MACH 3xx/4xx devices only) The clock under consideration is used to clock a macrocell to which an equation of 19 or more product terms, after minimization and including the XOR product term, is assigned.

To force a single-literal clock signal to be configured as a global clock, follow these steps:

1.	Preplace the signal at one of the global clock pins.

2.	Remove any conditions from this signal that would force the signal to be non-global.  (Any such condition would conflict with step 1, above, and result in a compiler error.)

3.	For devices other than the MACH 1xx/2xx and MACH465 devices (for which the option is not available): set the Global clocks routable as PT clocks? option of the MACH Fitting Options form to "N."

## Controlling Set and ResetXE "Set:controlling"§XE ".SETF"§XE "Reset:controlling"§XE ".RSTF"§

Use .SETF and .RSTF functional equations to control the preset and reset functions of flip-flops.  The general forms for .SETF and .RSTF functional equations are shown below.

Pin_name.SETF = Pin_or_product_term

Pin_name.RSTF = Pin_or_product_term

When the .SETF and .RSTF equations for the same signal share identical or equivalent terms, they are said to "overlap" (meaning that it is possible that both conditions can be true at the same time). Be careful to avoid overlapping Set and Rest conditions in your designs.

The Fitter will not assign any signal with either a Set or a Reset condition, but not both, to a block if such an assignment would cause that signal to inherit a Set or Reset condition that contains a term in common with the Set or Rest signal it already has.  This is true even if the SET/RESET treated as DONT CARE option in the Logic Synthesis Options form is set to "Y."  Refer to "Set/Reset Signals" in Chapter 8, "Using the Fitter," for additional information.

### Sharing Set and ResetXE "sharing Set and Reset"§XE "Set:sharing"§XE "Reset:sharing"§

In the MACH 1xx/2xx (except MACH215) devices:
Macrocells within a block share common set and reset signals.

In the MACH 215 and MACH 3xx/4xx devices:

r	All synchronous macrocells in the same PAL block share a common block Set line and a common block Reset line.  The block Set line is controlled by a single product term, as is the block Reset line.

r	Each asynchronous macrocell has an individual product term that can be used for either Set or Reset.

| ã | Note: For details on how specific MACH devices handle set and reset, refer to the device data book and Chapter 10, "Device Reference." |
| --- | --- |

The following example shows how pins A[0] through A[3] are reset whenever the signal RST is high.

In a MACH 3xx/4xx device, pins A[4] through A[7], although they share the same clock signal used by pins A[0] through A[3], are not affected when RST is asserted.  In a MACH 1xx/2xx device, pins A[4] through A[7] inherit the same reset signals used by pins A[1] through A[3].

;------------PIN Declarations ----------------------------
PIN	?	RST				;INPUT 
PIN	?	IN				;INPUT
PIN	?	A[7..0]		REGISTERED	;OUTPUT
PIN	?	CLK				;CLOCK
GROUP  MACH_SEG_A   A_GROUP  A[7..0]

;------------Boolean Equation Segment --------------------
EQUATIONS
A[3..0].RSTF = RST

A[7..0].CLKF = CLK

| ã | Note: This behavior described in the previous example occurs only if the Set/Reset treated as DON'T CARE option on the MACH Fitter Options form is set to "N" (No). Refer to the section titled "Set/Reset Compatibility" in Chapter 10 for details. |
| --- | --- |

# VectorsXE "vectors"§

A vector is a specific set of signals (inputs, outputs, or internal nodes) in which the order of the signals is constant.  The most common type of vector is a range of pins or nodes, but you can use comma-delimited vectors in some language constructs.

## Ranges of Pins or NodesXE "ranges of pins or nodes"§

A range is a set of pins or nodes that have the same root name.  Members of the range are differentiated by subscript.  For example, in the range NAME[1..5], the members are:

NAME[1]
NAME[2]
NAME[3]
NAME[4]
NAME[5]

Ranges are declared in pin and node statements and referenced in other statements.

You must observe the following rules for range notation.

r	Reference individual pins or nodes in a range using subscripted pin or node names.  Use the format NAME[1] rather than NAME1.

r	Reference groups of pins or nodes in the range using the range operator (NAME[1..4]) or by separating individual signal subscripts with commas (NAME[1..4,5,7]).

You can include input and output pins in the same range if your application calls for it, but you cannot include pins and nodes in the same range.  Define ranges of contiguous pins by separating the first and last members with two periods.  For example, you specify the range of input pins 3 through 6 as follows.

3..6

To include non-contiguous pin numbers, you must separate them using commas:

1..4, 8..11

In the pin name field, enter the range name followed by a range that indicates the desired subscripts.  Enclose the range in square brackets as follows.

NAME[1..4]

Enter the pin numbers using range notation as indicated in the next example.

PIN   3..6   NAME[1..4]   COMBINATORIAL

You can use a single question mark, ?, to float the location of all the pins in the range as shown in the next example.

PIN      ?   NAME[1..4]   COMBINATORIAL

Enter the polarity and storage type attributes for the range as you would for a single pin.

| ã | Note: Vectors created using a range of pins or nodes can be referenced throughout the design as complete vectors ( NAME[1..4] ) or as individual members ( NAME[1] and NAME[2] ). Comma-delimited vectors, described below, cannot be referenced elsewhere; they are used within a single CASE or IF-THEN-ELSE statement only. Vectors are illegal in the CONDITIONS portion of the STATE segment (if any). |
| --- | --- |

## Comma-Delimited VectorsXE "comma-delimited vectors"§XE "vectors:comma-delimited"§

The comma-delimited vector is used in a single CASE or IF-THEN-ELSE statement and cannot be referenced elsewhere in the design.  It takes the following general form:

Name_1, Name_2, Name_3, ... ,Name_

The order of signals within a comma-delimited vector remains fixed, as is the case with a vector declared using a range.  For example, both of the following vectors can be used to represent four-bit binary numbers:

| Range Vector | Comma-delimited Vector |
| --- | --- |
| NAME[1..4] | BIT1, BIT2, BIT3, BIT4 |

Furthermore, a comma-delimited vector can include range vectors, as shown here:

PIN1, PIN2, NAME[1..4], PIN7, NAME[8]

Using the flexibility of the comma-delimited vector, you might, for example, declare several four-bit ranges and consider them in different orders, as shown below:

| Different Groupings of Three Four-Bit Ranges |  |
| --- | --- |
| Comma-delimited vector 1: | NAME[1..4], NAME[5..8], NAME[9..12] |
| Comma-delimited vector 2: | NAME[1..4], NAME[9..12], NAME[5..8] |
| Comma-delimited vector 3: | NAME[5..8], NAME[1..4], NAME[9..12] |
| Comma-delimited vector 4: | NAME[5..8], NAME[9..12], NAME[1..4] |
| Comma-delimited vector 5: | NAME[9..12], NAME[1..4], NAME[5..8] |
| Comma-delimited vector 6: | NAME[9..12], NAME[5..8], NAME[1..4] |

# Radix OperatorsXE "radix operators"§

A radix is a construct used on the right side of an equation, in an IF..THEN..ELSE statement, or in a CASE statement, to represent a number in binary, octal, decimal, or hexadecimal format. The radix is converted automatically to a binary bit pattern, which is then compared with a vector of pin or node values on the left side of the equation.

The decimal radix (base 10) is the default for CASE statements.  You can also use binary, octal, or hexadecimal radices (base 2, 8, and 16, respectively).

To use a radix other than the default, you must precede the test condition with the appropriate radix operator.  The table below shows the radix operators for all four radices supported by the software.

| Operators | Definitions |
| --- | --- |
| #bXE "#b"§ or #B | Specifies the binary radixXE "binary radix"§, base 2 |
| #oXE "#o"§ or #O | Specifies the octal radixXE "octal radix"§, base 8 |
| #dXE "#d"§ or #D | Specifies the decimal radixXE "decimal radix"§, base 10 (default) |
| #hXE "#h"§ or #H | Specifies the hexadecimal radixXE "hexadecimal radix"§, base 16 |

If you omit the radix operator altogether, the default, which is decimal form, is assigned.

Examples of each are shown below.

| Syntax | Function |
| --- | --- |
| #b1011 or #B1011 | ;represents 1011 (10112) |
| #o13 or #O13 | ;represents 1011 (138) |
| 11 or #d11 or #D11 | ;represents 1011 (1110) |
| #hB or #HB | ;represents 1011 (B16) |

| ã | Note: When radix notation is used in CASE statements and Boolean equations, numbers that are equal to or greater than 16000 must be expressed in hexadecimal form. Smaller numbers can be expressed in other radices. |
| --- | --- |

When you use radix notation in a statement, it is automatically expanded to its binary equivalent and compared to the vector specified on the left side of the equation.  If the binary equivalent does not have enough digits, leading zeros are added during processing as required.

The first number in the vector is the most significant bit.  For example, in the vector ADDRESS[3..0], ADDRESS[3] is the most significant bit.

When the example below is used in an IF..THEN..ELSE construct, the radix on the right hand side of the equation is compared to the vector on the left.

ADDRESS[3..0] = 3

The binary equivalent of 310 is 112.  Since the vector ADDRESS[3..0] contains four signals, the binary number must be "padded" with two zeros on the left (like this: 0011) so that each signal in the vector has a value.

The four signals in the vector are compared to their corresponding bits in the expanded radix as indicated below.

r	ADDRESS[3] compared to 0

r	ADDRESS[2] compared to 0

r	ADDRESS[1] compared to 1

r	ADDRESS[0] compared to 1

If all four conditions evaluate true, the equation is true.

Important
When comparing a vector to a radix, be careful to specify the order of the least and most significant bits correctly.  For example, the first line gives different results than the second.

ADDRESS[3..0] = 3
ADDRESS[0..3] = 3

# IF-THEN-ELSE StatementsXE "IF-THEN-ELSE statements"§

The IF-THEN-ELSE statement is a flow-of-control construct that expresses logical operations in natural language.  You can use this construct as an alternative to writing Boolean equations.

The syntax for the IF-THEN-ELSE statement is:

IF Test condition THEN

BEGIN
		Action(s)	;performed if test condition = true
	END
ELSE
	BEGIN
		Action(s)	;performed if test condition = false
	END
If you do not specify the else condition, it is treated as a don't care when the logic is generated (if the Use 'IF-THEN-ELSE', 'CASE' default as option in the Logic Synthesis Options form is set to "Don't Care.")

The following example shows testing the high order bit on an 8-bit address line.  If it is equal to 1, the signal named HIBIT is set high and LO_BANK_ENA is set low.  If it is equal to 0, HIBIT is set low and LO_BANK_ENA is set high.

;------------ PIN Declarations ---------------------------
PIN     ?     ADDRESS[7..0]		;INPUT
PIN     ?     HIBIT	REGISTERED	;OUTPUT
PIN     ?     LO_BANK_ENA	REGISTERED	;OUTPUT
PIN     ?     CLK		;CLOCK

Continued...

...Continued

;------------ Boolean Equation Segment -------------------
EQUATIONS
IF ADDRESS[7] = 1 THEN
  BEGIN
    HIBIT = 1
    LO_BANK_ENA = 0
  END

ELSE
  BEGIN
    HIBIT = 0
    LO_BANK_ENA = 1
  END
HIBIT.CLKF = CLK
LO_BANK_ENA.CLKF = CLK

| ã | Note: Refer to "The "Don't Care" Logic-Synthesis Option" section, under "CASE Statements," for information on how the "Don't Care" option affects CASE and IF-THEN-ELSE logic. |
| --- | --- |

# CASE StatementsXE "CASE statements"§

The CASE statement is a flow-of-control construct that is useful for testing a number of different conditions. The syntax for the CASE statement is as follows.

CASE (Condition_signals)
BEGIN
	Value:
		BEGIN
			Action
		END
...
	OTHERWISE:
		BEGIN
			Action
		END
  END

The following example asserts enable lines for four peripheral devices named UNIT1 through UNIT4 by checking for their hexadecimal address on an 8-bit address line.  The declarations are shown first.

;-------------PIN Declarations ---------------------------
PIN     ?     ADD[7..0]                 ;INPUT
PIN     ?     UNIT1      REGISTERED     ;OUTPUT
PIN     ?     UNIT2      REGISTERED     ;OUTPUT
PIN     ?     UNIT3      REGISTERED     ;OUTPUT
PIN     ?     UNIT4      REGISTERED     ;OUTPUT
PIN     ?     CLK                       ;CLOCK

;A special test condition, indicated by a value of 0, 1, 2 
;or 3 on the address bus, is checked.  If this condition 
;is detected, all four enable lines are asserted.  The 
;range notation is used to test for this condition, which 
;results in a more compact notation.

;-------------Boolean Equation Segment -------------------
EQUATIONS

CASE (ADD[7..0])
  BEGIN
	#h0F:
		BEGIN
			UNIT1 = 1
			UNIT2 = 0
			UNIT3 = 0
			UNIT4 = 0
		END
	#h2F:

BEGIN
			UNIT1 = 0
			UNIT2 = 1
			UNIT3 = 0
			UNIT4 = 0
		END
	#h5F:
		BEGIN
			UNIT1 = 0
			UNIT2 = 0
			UNIT3 = 1
			UNIT4 = 0
		END
	#hFF:
		BEGIN
			UNIT1 = 0
			UNIT2 = 0
			UNIT3 = 0
			UNIT4 = 1

END

Continued...

...Continued

OTHERWISE:
		BEGIN
			UNIT1 = 0
			UNIT2 = 0
			UNIT3 = 0
			UNIT4 = 0
		END
          END

UNIT1.CLKF = CLK
UNIT2.CLKF = CLK
UNIT3.CLKF = CLK
UNIT4.CLKF = CLK

## Building State Machines XE "state machines:building with CASE "§with CASEXE "CASE:building state machines"§ Statements

The CASE statement is the preferred way to implement state machines in the MACHXL software syntax.  The following example implements the traffic light controller state machine shown in the following state diagram.

µ §

This design uses the following pin statements:

TITLE		MULTIPLE STATE MACHINES

PATTERN		A

REVISION	1.0

AUTHOR          J. ENGINEER

COMPANY		ADVANCED MICRO DEVICES

DATE		02/12/93

CHIP	MULTISTATE   MACH435

;============================================================

;            TRAFFIC CONTROLLER PIN DEFINITIONS

;============================================================

PIN ?  CLOCK1 ; CLOCK

PIN ?  SENSOR ; INPUT

; Outputs for controlling signals in the traffic example

;	VOUT[5]  VOUT[4]  VOUT[3]  VOUT[2]  VOUT[1]

;	RED1  YELLOW1  GREEN1  RED2  YELLOW2  GREEN2

PIN ?  VOUT[5..0] REGISTERED ; OUTPUTS

PIN ?  ST[1,0] REGISTERED ; STATE BITS

Both CASE and IF-THEN-ELSE statements allow you to evaluate binary, octal, decimal, or hexadecimal numbers using vectors of signals.  The pins statements above define two vectors:

| VOUT[5..0] | Defines the six outputs that control the operation of six lamps. Throughout the design, these lamps are referred to as RED1 (VOUT[5]), YELLOW1 (VOUT[4]), GREEN1 (VOUT[3]), RED2 (VOUT[2]), YELLOW2 (VOUT[1]), and GREEN2 (VOUT[0]). |
| --- | --- |
| ST[1,0] | Defines the two state bits required to define the four states used in the traffic controller design. |

The STRING statement simplifies the job of writing state machine designs using CASE statements, and makes it much easier for a reader to understand the design.

You can use STRING statements to do the following:

r	Assign logical names to the state-bit values that correspond to each state

r	Define the output values of each state (or each branch, if your design is a Mealy machine)

r	Represent the state bits themselves, to make it easier to assign them new values (to branch to a different state)

The following statements define strings that help manage the traffic controller state machine, including the four states: 1GREEN, 1YELLOW, 2GREEN, and 2YELLOW.

;============================================================

;            TRAFFIC CONTROLLER STRING DEFINITIONS

;============================================================

; state assignments

STRING	1GREEN		'0'

STRING	1YELLOW		'1'

STRING	2GREEN		'3'

STRING	2YELLOW		'2'

STRING	M1		'ST[1,0]'

STRING	V		'VOUT[5..0]'

; output vector definitions

; VOUT[5]  VOUT[4]  VOUT[3]  VOUT[2]  VOUT[1]  VOUT[0]

; RED1  YELLOW1  GREEN1  RED2  YELLOW2  GREEN2

STRING	G1_VECT		'#B001100'

STRING	Y1_VECT		'#B010100'

STRING	G2_VECT		'#B100001'

STRING	Y2_VECT		'#B100010'

Remember to clock the registers used in your design:

EQUATIONS

;============================================================

;            TRAFFIC CONTROLLER EQUATIONS

;============================================================

VOUT[5..0].CLKF=CLOCK1

VOUT[5..0].RSTF =_RESET1

ST[1,0].CLKF=CLOCK1

ST[1..0].RSTF =_RESET1

The general form of a state machine implementation is:

CASE (State_bits)
BEGIN
	Value_of_state_1:
		BEGIN
			Assign values to output pins
			Assign new state's values to state bits
		END
	Value_of_state_2:
		BEGIN
			Assign values to output pins
			Assign new state's values to state bits
		END
	...
	Value_of_state_n:
		BEGIN
			Assign values to output pins
			Assign new state's values to state bits
		END
END

The CASE statements used to implement the traffic controller design are shown below.

CASE (M1)

BEGIN

1GREEN:

BEGIN

IF (SENSOR) THEN

BEGIN

V=Y1_VECT

M1=1YELLOW

END

ELSE

BEGIN

V=G1_VECT

M1=1GREEN

END

END

1YELLOW:

BEGIN

V=G2_VECT

M1=2GREEN

END

2GREEN:

BEGIN

IF (SENSOR) THEN

BEGIN

V=Y2_VECT

M1=2GREEN

END

ELSE

BEGIN

V=G2_VECT

M1=2GREEN

END

END

2YELLOW:

BEGIN

V=G1_VECT

M1=1GREEN

END

END

When you simulate the design, use the pins themselves (not the logical names you assigned them) as shown below.

SIMULATION

;============================================================

;            SIMULATION FOR TRAFFIC CONTROLLER

;============================================================

TRACE_ON CLOCK1 SENSOR ST[1,0] VOUT[5..0]

SETF    _RESET1 /CLOCK1  ; RESET REGISTERS

SETF    /_RESET1

CHECK   VOUT[0] VOUT[1] VOUT[5] /VOUT[4] VOUT[3] /VOUT[2]

/ST[0]  ST[1]

SETF    /SENSOR

CLOCKF   CLOCK1

CHECK   /VOUT[0] /VOUT[1] /VOUT[5] /VOUT[4] VOUT[3] VOUT[2]

/ST[0]   /ST[1]

SETF     SENSOR

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    /VOUT[0] VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

SETF    /SENSOR

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    VOUT[0] /VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    VOUT[0] /VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

TRACE_OFF

## Multiple State MachinesXE "multiple state machines"§XE "state machines:multiple"§

This section shows how to combine two state machine designs in a single device.  You can combine as many state machines as you like, subject only to the availability of resources in the target device.

The two state machines in the design—a traffic controller and an answering machine—were selected because they are common examples of state machines and hence require no explanation.

| ã | Note: In the example design that follows, the two state machines are completely independent of one another. However, you can implement state machines that interact with one another. To do this, use the state bits and/or outputs of one state machine as inputs to another state machine. |
| --- | --- |

The example is structured as follows: each state machine's pin declarations are entered first, followed by the string definitions for each.  Next, the equations for each state machine are entered.  Finally, the simulation statements for each state machine are entered.  The procedures you follow to enter multiple state machine designs are identical to those you follow for single state machines.

Observe the following rules:

r	Avoid duplicate names for state bits and output vectors (in this example, the state bits for one state machine are called M1 and the state bits for the other M2).

r	Avoid manipulating the wrong state machine's state bits or outputs.  (For example, you might type M2 instead of M1 and set the second state machine's state bits from within the CASE statement for the first state machine.  Such an error is very difficult to debug.)

TITLE		MULTIPLE STATE MACHINES

PATTERN		A

REVISION	1.0

AUTHOR          J. ENGINEER

COMPANY		ADVANCED MICRO DEVICES

DATE		02/12/93

CHIP	MULTISTATE   MACH435

;============================================================

;            TRAFFIC CONTROLLER PIN DEFINITIONS

;============================================================

PIN ?  CLOCK1 ; CLOCK

PIN ?  SENSOR ; INPUT

; Outputs for controlling signals in the traffic example

;	VOUT[5]  VOUT[4]  VOUT[3]  VOUT[2]  VOUT[1]

;	RED1  YELLOW1  GREEN1  RED2  YELLOW2  GREEN2

PIN ?  VOUT[5..0] REGISTERED ; OUTPUTS

PIN ?  ST[1,0] REGISTERED ; STATE BITS

;============================================================

;            ANSWERING MACHINE PIN DEFINITIONS

;============================================================

PIN ?  CLOCK2 ; CLOCK

PIN ?  _RESET1 ; RESET CONTROL

PIN ?  _RESET2 ; RESET CONTROL

PIN ?  RING ; INPUTS

PIN ?  ENDGREETING

PIN ?  DIALTONE

PIN ?  ENDMESSAGES

PIN ?  ST2[1,0] REGISTERED ; STATE BITS

PIN ?  ANSWER REGISTERED ; OUTPUTS

PIN ?  PLAY REGISTERED

PIN ?  RECORD REGISTERED

GROUP	M2REG	ST2[1,0]  ANSWER  PLAY  RECORD	; GLOBAL USE

;============================================================

;            TRAFFIC CONTROLLER STRING DEFINITIONS

;============================================================

; state assignments

STRING	1GREEN		'0'

STRING	1YELLOW		'1'

STRING	2GREEN		'3'

Continued...

...Continued

STRING	2YELLOW		'2'

STRING	M1		'ST[1,0]'

STRING	V		'VOUT[5..0]'

; output vector definitions

; VOUT[5]  VOUT[4]  VOUT[3]  VOUT[2]  VOUT[1]  VOUT[0]

; RED1  YELLOW1  GREEN1  RED2  YELLOW2  GREEN2

STRING	G1_VECT		'#B001100'

STRING	Y1_VECT		'#B010100'

STRING	G2_VECT		'#B100001'

STRING	Y2_VECT		'#B100010'

;============================================================

;            ANSWERING MACHINE STRING DEFINITIONS

;============================================================

; state assignments

STRING	WAITING		'0'

STRING	PLAYING		'1'

STRING	RECORDING	'2'

STRING	M2NULL		'3'

STRING	M2		'ST2[1,0]'

;------------------------------------------------------------

EQUATIONS

;============================================================

;            TRAFFIC CONTROLLER EQUATIONS

;============================================================

VOUT[5..0].CLKF=CLOCK1

VOUT[5..0].RSTF =_RESET1

ST[1,0].CLKF=CLOCK1

ST[1..0].RSTF =_RESET1

CASE (M1)

BEGIN

1GREEN:

BEGIN

IF (SENSOR) THEN

BEGIN

V=Y1_VECT

Continued...

...Continued

M1=1YELLOW

END

ELSE

BEGIN

V=G1_VECT

M1=1GREEN

END

END

1YELLOW:

BEGIN

V=G2_VECT

M1=2GREEN

END

2GREEN:

BEGIN

IF (SENSOR) THEN

BEGIN

V=Y2_VECT

M1=2GREEN

END

ELSE

BEGIN

V=G2_VECT

M1=2GREEN

END

END

2YELLOW:

BEGIN

V=G1_VECT

M1=1GREEN

END

END

;============================================================

;            ANSWERING MACHINE EQUATIONS

;============================================================

M2REG.CLKF=CLOCK2

M2REG.RSTF=_RESET2

Continued...

...Continued

CASE (M2)

BEGIN

WAITING:

BEGIN

IF (RING) THEN

BEGIN

ANSWER=1

PLAY=1

RECORD=0

M2=PLAYING

END

ELSE

BEGIN

ANSWER=0

PLAY=0

RECORD=0

M2=WAITING

END

END

PLAYING:

BEGIN

IF (DIALTONE) THEN

BEGIN

ANSWER=0

PLAY=0

RECORD=0

M2=WAITING

END

IF (/DIALTONE * /ENDGREETING) THEN

BEGIN

ANSWER=1

PLAY=1

RECORD=0

M2=PLAYING

END

IF (/DIALTONE * ENDGREETING) THEN

BEGIN

ANSWER=1

PLAY=0

RECORD=1

M2=RECORDING

END

END

Continued...

...Continued

RECORDING:

BEGIN

IF (/ENDMESSAGES * /DIALTONE) THEN

BEGIN

ANSWER=1

PLAY=0

RECORD=1

M2=RECORDING

END

IF (ENDMESSAGES + DIALTONE) THEN

BEGIN

ANSWER=0

PLAY=0

RECORD=0

M2=WAITING

END

END

M2NULL:

BEGIN

M2=WAITING

ANSWER=0

PLAY=0

RECORD=0

END

END

;------------------------------------------------------------

SIMULATION

;============================================================

;            SIMULATION FOR TRAFFIC CONTROLLER

;============================================================

TRACE_ON CLOCK1 SENSOR ST[1,0] VOUT[5..0]

SETF    _RESET1 /CLOCK1  ; RESET REGISTERS

SETF    /_RESET1

CHECK   VOUT[0] VOUT[1] VOUT[5] /VOUT[4] VOUT[3] /VOUT[2]

/ST[0]  ST[1]

SETF    /SENSOR

CLOCKF   CLOCK1

CHECK   /VOUT[0] /VOUT[1] /VOUT[5] /VOUT[4] VOUT[3] VOUT[2]

/ST[0]   /ST[1]

Continued...

...Continued

SETF     SENSOR

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    /VOUT[0] VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

SETF    /SENSOR

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    VOUT[0] /VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CLOCKF   CLOCK1

CHECK    VOUT[0] /VOUT[1] VOUT[5] /VOUT[4] /VOUT[3] /VOUT[2]

ST[0]  ST[1]

TRACE_OFF

;============================================================

;            SIMULATION FOR ANSWERING MACHINE

;============================================================

TRACE_ON CLOCK2 RING ENDGREETING DIALTONE ENDMESSAGES ST2[1,0]

ANSWER PLAY RECORD

SETF    _RESET2 /CLOCK2  ; RESET REGISTERS

SETF    /_RESET2

CHECK   /PLAY /ANSWER  RECORD

/ST2[0] ST2[1]

SETF    /ENDMESSAGES /DIALTONE /ENDGREETING /RING

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK   /PLAY ANSWER RECORD

/ST2[0] ST2[1]

SETF     RING

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK   /PLAY  ANSWER  RECORD

/ST2[0] ST2[1]

Continued...

...Continued

SETF    /RING

CLOCKF   CLOCK2

CHECK   /PLAY  ANSWER  RECORD

/ST2[0] ST2[1]

SETF     DIALTONE

CLOCKF   CLOCK2

CHECK   /PLAY  /ANSWER  /RECORD

/ST2[0] /ST2[1]

SETF    /DIALTONE

SETF     RING

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK    PLAY  ANSWER  /RECORD

ST2[0] /ST2[1]

SETF     ENDGREETING

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK   /PLAY  ANSWER  RECORD

/ST2[0] ST2[1]

SETF    /ENDGREETING ENDMESSAGES /RING

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK   /PLAY  /ANSWER  /RECORD

/ST2[0] /ST2[1]

SETF    /ENDMESSAGES

CLOCKF   CLOCK2

CLOCKF   CLOCK2

CHECK   /PLAY  /ANSWER  /RECORD

/ST2[0] /ST2[1]

TRACE_OFF

;============================================================

;            END OF DESIGN

;============================================================

## The "Don't-Care" Logic-Synthesis OptionXE "don't care logic-synthesis option"§

When processing a design file with IF-THEN and CASE statements, the MACHXL software provides a logic synthesizing option to process unspecified states as Don't Cares, or to set unspecified states to 0 (i.e., turn OFF these states). This option, Use 'IF-THEN-ELSE', 'CASE' default as appears on the Logic Synthesis Options form.  The available settings ("Don't Care" and "Off") affect the logic created and also the operation of the device.

To see how the Use 'IF-THEN-ELSE', 'CASE' default as option affects designs you write, consider the following example.

Design Specification
Lights shall be turned on 
	a) when someone is in the room and 
	b) when it is night.

If you specify such behavior verbally, your expectation is that the listener will turn on the lights when he or she is in the room at night, and that the listener will turn off the lights at all other times.  Your normal expectation corresponds to the "Off" setting of the Use 'IF-THEN-ELSE', 'CASE' default as option.  The "Off" setting means that the software assumes all signals are off in all unspecified conditions.

The design example below translates the design specification ABOVE into MACHXL syntax.

TITLE      DONT_CARE_TEST

PATTERN    A

REVISION   1.0

AUTHOR     J. ENGINEER

COMPANY    AMD

DATE       03/01/93

CHIP    DONT_CARE_TEST    MACH435

pin     4       in_room                 ;input

pin     3       night                   ;input

pin     8       light_on                ;output

equations

if (in_room * night) then

begin

light_on = VCC		;on cover is:

end				;light_on = in_room * night

if (/in_room) then

begin

light_on = GND		;off cover is

end				;/light_on = /in_room

The disassembled intermediate file BELOW shows how the MACHXL software implements the design example, above, with the option set to "Off."

TITLE    DONT_CARE_TEST   DIS-ASSEMBLED

PATTERN  001

REVISION 001

AUTHOR   J.ENGINEER

COMPANY  AMD

DATE     Fri Oct 07 14:26:59 1994

CHIP     UNSPECIFIED   MACH435

PIN    20         CK0_

PIN    23         CK1_

PIN    62         CK2_

PIN    65         CK3_

PIN     3        NIGHT

PIN     4      IN_ROOM

PIN     8     LIGHT_ON

NODE    2          R2_      PAIR     LIGHT_ON

EQUATIONS

LIGHT_ON       = IN_ROOM * NIGHT

R2_            = {LIGHT_ON}

The behavior of this disassembled design is the same as you would expect from a human listener: the lights are on ONLY if both of the following inputs are true:  IN_ROOM and NIGHT.

By contrast, when the Use 'IF-THEN-ELSE', 'CASE' default as option is set to "Don't Care," the same design file, after compilation, disassembles as shown below.

; COMPILED WITH OPTION SET TO "DON'T CARE"

TITLE    DONT_CARE_TEST   DIS-ASSEMBLED

PATTERN  001

REVISION 001

AUTHOR   J.ENGINEER

COMPANY  AMD

DATE     Fri Oct 07 14:29:27 1994

CHIP     UNSPECIFIED   MACH435

PIN    20         CK0_

Continued...

...Continued

PIN    23         CK1_

PIN    62         CK2_

PIN    65         CK3_

PIN     4      IN_ROOM

PIN     8     LIGHT_ON

NODE    2          R2_      PAIR     LIGHT_ON

EQUATIONS

LIGHT_ON       = IN_ROOM

R2_            = {LIGHT_ON}

The behavior of this disassembled design is NOT the same as you would expect from a human listener: the state of the lights is a function of IN_ROOM alone, and the input condition NIGHT is ignored altogether.  The reason for this behavior can be understood by examining the Karnaugh map of the design as it exists just before logic minimization.  The symbol "X" (Don't Care) marks the location of the condition that was not specified in the original design.

µ §

When the Use 'IF-THEN-ELSE,' 'CASE' default as option is set to "Off," all "X" symbols in the Karnaugh map are replaced with zeroes, as shown below.  Given this Karnaugh map, the Logic Minimizer cannot eliminate either input from the equation for LIGHTS_ON, so the behavior remains as specified.

µ §

When the Use 'IF-THEN-ELSE,' 'CASE' default as option is set to "Don't Care," however, the Logic Minimizer can group the "X" symbol with either an adjacent 1 or 0, as required to minimize the equation.

The Karnaugh map below shows how the Logic Minimizer groups the "X" with 1.

µ §

This produces the equation

LIGHTS_ON = IN_ROOM * /NIGHT + IN_ROOM * NIGHT

which minimizes to

LIGHT_ON = IN_ROOM.

# MINIMIZE_ON XE "MINIMIZE_ON "§and MINIMIZE_OFFXE "MINIMIZE_OFF"§

Place a pair of MINIMIZE_OFF and MINIMIZE_ON statements in the PDS file around any portion of the design you do not want minimized.  (The MINIMIZE_ON statement is only required if the design file contains subsequent statements that you want minimized.)

For example, to retain the redundant product terms that are necessary to prevent race hazards in equations used to emulate latch behavior, insert a MINIMIZE_OFF statement before the set of equations.

If the MINIMIZE_OFF precedes an equation, the Logic Minimizer does not perform logic reduction, although it may perform other operations.  For example, if the equation is not expressed as a sum of products, the Logic Minimizer does remove parentheses and apply DeMorgan's theorem as required.

Even if preceded by MINIMIZE_OFF, the equation

OUT1 = /(X + Y) + /X * Y

will be expanded by the Logic Minimizer to

OUT1 = /X * /Y + /X * Y

However, unless the Logic Minimizer is allowed to operate fully on the equation, it will not be reduced to its minimal form:

OUT1 = /X

There are two cases in which the Minimizer must reconcile ambiguities:

r	If the MINIMIZE_OFF keyword separates the equations for paired pins and nodes, the pin equation takes precedence.  That is, if the pin equation follows MINIMIZE_OFF, neither the pin nor the node equation will be minimized (even if the node equation precedes MINIMIZE_OFF), and if the pin equation precedes MINIMIZE_OFF, both the pin and the node equations will be minimized.

r	If the MINIMIZE_OFF keyword separates the on and off covers for the same signal, both are treated the same way with respect to minimization and the signal that appears last in the design file determines how both signals are treated.

Example

SIG2 = X

MINIMIZE_OFF

/SIG2 = X * Y	;SIG2 and /SIG2 equations are not minimized

The MACH215 and MACH4xx devices have dedicated input registers as well as output registers.  Other MACH2xx devices have output registers to which input signals can be routed without passing through the sum-or-products array and incurring the corresponding propagation delay (however note that each registered input decreases the number of registers available for output or buried feedback by one).  MACH1xx devices have no input registers.

MACH1xx devices do not have hardware latches.  You can implement latches using combinatorial logic, as described in the "Latches" section of Chapter 10, "Device Reference."

Radix operators can be used in SIGNATURE statements to define JTAG user codes.  Rules for using radix notation in JTAG user codes differ from those that apply to CASE statements and Boolean equations.  Refer to SIGNATURE in Chapter 5 for more information.