| 9 | Report Files |
| --- | --- |

Contents

µOverview	311

Log File	312

Fitting Report	318

Header Information	318

MACH Fitter Options	318

Device Resource Summary	320

Block Partitioning Summary	322

Signal Summary	324

PRESET, RESET and OUTPUT ENABLED 
Signal Summary	327

Tabular Information	328

Fitting Status	338

Place and Route Data Report	339

Unplaceable Designs	340

Unroutable Designs	340

Place and route processing time	341

Place/Route Resource and Usage tables	341

Signal Fan-Out Table	343

Device pin-out list	344

Block information	345

Macrocell (MCell) Cluster Assignments	345

Maximum PT Capacity	350

Node-Pin Assignments	351

IO-to-Node Pin Mapping	353

IO/Node and IO/Input Macrocell Pairing Table	356

Input and Central switch matrix tables	357

Input Multiplexer (IMX) Assignments	357

Logic Array Fan-in	359

Using Place and Route Data to Limit Placements	362

Timing Analysis Report	364

TSU	366

TCO	367

TPD	368

TCR	369

Failure Reports	370

Failure to Partition	370

Failure to Place	372

Failure to Route	373

# Overview

The log file, Design.LOG, is created when you compile a design.  Each compilation and fitting program appends its own log data to the log file.  (The reports generated separately by the Fitter contain detailed partitioning, placement, and routing information.  Refer to "Fitting Report," "Place and Route Data Report," and "Timing Analysis Report," in this chapter, for details.

When the design is successfully processed, the Fitter supplies the user with three additional reports:

r	The Fitter report (Design.RPT) contains the following information:

u	A summary of the Fitter options used during the current fitting pass

u	Resource and utilization information

u	Block partitioning information in the form of summary of resources used by each block

u	Information about placements, fanout and fanin of signals to a block

u	Output pin-to-node pairing

r	The Place and Route report (Design.PRD) contains the following information:

u	Place and route processing time

u	Place/Route Resource and Usage tables

u	Signal fan-out table sorted in alphabetical order

u	Device pin-out list

u	Block information

u	Input and Central switch matrix tables

r	The Timing analysis report (Design.TAL) gives a timing analysis for all signals

When the Fitter fails to complete processing a design, it generates partial reports.  These reports are described in the "Failure Reports" section of this chapter.

# Log File

The log file begins by displaying the menu options in effect when the design was processed, as shown below.

COMPILATION OPTIONS

Log file name:    BAR_4XX.log

Run mode:         Run All Programs

Format:  Text     File:  BAR_4XX.PDS

MACH FITTING OPTIONS

SIGNAL PLACEMENT:

Handling of Preplacements                 No Change

Use placement data from                   Design file

Save last successful placement            <F3>

Press <F9> to edit file containing        Last successful placement

FITTING OPTIONS:

Global Clocks routable as Pterm Clocks?   N

22V10/MACH1XX/2XX S/R Compatibility?      Y

SET/RESET treated as DONT_CARE?           N

Run Time Upper Bound in 15 minutes        1

Iterate between partition & place/route?  Y

Balanced partitioning?                    N

Spread placement?                         Y

Reduce Non-forced Global Clocks?          N  .. if 'Y', Number = 1

Reduce Routes Per Placement?              N

LOGIC SYNTHESIS OPTIONS

Use automatic pin/node input pairing?  N

Use automatic gate splitting?          N  .. if 'Y', Threshold = 20

Gate split max num. pterms per eqn     20

Optimize registers for D/T-type        Change all to D-type

Ensure polarity after minimization is  Best for device

Use 'IF-THEN-ELSE','CASE' default as   Don't care

Use fast minimization?                 N

Then, each program module invoked provides details of its own processing.

Error messages explain why processing failed.  If the error is reported by the Parser, the log file shows the location of the error in the design file, as shown in the file fragment below, in which the pin number or float operator (?) was omitted from line 14.

COMPILATION OPTIONS

Log file name:    bar_4xx.log

Run mode:         Run All Programs

Format:  Text     File:  BAR_4XX.PDS

MACH FITTING OPTIONS

SIGNAL PLACEMENT:

Handling of Preplacements                 No Change

Use placement data from                   Design file

Save last successful placement            <F3>

Press <F9> to edit file containing        Last successful placement

FITTING OPTIONS:

Global Clocks routable as Pterm Clocks?   N

22V10/MACH1XX/2XX S/R Compatibility?      Y

SET/RESET treated as DONT_CARE?           N

Run Time Upper Bound in 15 minutes        1

Iterate between partition & place/route?  Y

bar_4xx.pds

MACHXL 2.0 R6 PARSER (09-23-94)

(C) - COPYRIGHT ADVANCED MICRO DEVICES INC., 1993, 1994

***********************************

*     MACHXL PARSER LISTING       *

***********************************

LINE #   |----+----1----+----2----+----3----+----4----+----5----+----6----+

1     |

2     |;Barrel Shifter

3     |;Where Shift Registers shift bits only one position to the left or

4     |;to the right, Barrel Shifters can shift data a selectable number of positions

5     |;in one direction.

6     |

7     |

8     |

9     |;---------------------------------- Declaration Segment ------------

10     |TITLE    Barrel Shifter

11     |PATTERN  1

12     |REVISION 1

13     |AUTHOR   J. ENGINEER

Continued...

...Continued

14     |COMPANY  AMD

15     |DATE    9/10/94

16     |

17     |CHIP  Barrel  MACH435

18     |

19     |;---------------------------------- PIN Declarations --------------

20     |

21     |PIN  ?          DATA[0..3]

22     |PIN  ?          Q[0..3]                              REGISTERED ;

23     |PIN  ?          SEL1

24     |pin  ?          SEL2

25     |pin  ?          RESET

26     |pin  ?          CLK

27     |PIN  ENA

ERROR -------^ (L27/C6)

|> ERROR P55  Unexpected symbol ENA in malformed statement.

28     |

29     |EQUATIONS

30     |

31     |Q[0..3].RSTF=RESET

32     |Q[0..3].CLKF=CLK

33     |Q[0..3].TRST=ENA

34     |

35     |Q[0]:= /SEL1*/SEL2*DATA[0]

36     |     +/SEL1* SEL2*Q[1]

37     |     + SEL1*/SEL2*Q[2]

38     |     + SEL1* SEL2*Q[3]

39     |

40     |Q[1]:= /SEL1*/SEL2*DATA[1]

41     |     +/SEL1* SEL2*Q[2]

42     |     + SEL1*/SEL2*Q[3]

43     |     + SEL1* SEL2*Q[0]

44     |

45     |Q[2]:= /SEL1*/SEL2*DATA[2]

46     |     +/SEL1* SEL2*Q[3]

47     |     + SEL1*/SEL2*Q[0]

48     |     + SEL1* SEL2*Q[1]

49     |

50     |Q[3]:= /SEL1*/SEL2*DATA[3]

51     |     +/SEL1* SEL2*Q[0]

52     |     + SEL1*/SEL2*Q[1]

53     |     + SEL1* SEL2*Q[2]

54     |

55     |

56     |SIMULATION

57     |

Continued...

...Continued

58     |TRACE_ON   data[3..0] q[3..0] sel1 sel2 clk

59     |

60     |SETF RESET  ena

61     |SETF DATA[3..0]= #H8

62     |SETF /RESET ena

63     |

64     |;---LOADING DATA

65     |SETF /SEL1 /SEL2

66     |CLOCKF CLK

67     |CHECKQ Q[3..0]= #H8

68     |

69     |;---    Shifting one position to the right, three times

70     |SETF /sel1 sel2

71     |FOR X:= 1 TO 3 DO

72     |        BEGIN

73     |             CLOCKF CLK

74     |        END

75     |CHECKQ Q[3..0]= #H1

76     |

77     |;---    Shifting two positions to the right, four times

78     |SETF  sel1 /sel2

79     |FOR X:= 1 TO 4 DO

80     |        BEGIN

81     |             CLOCKF CLK

82     |        END

83     |CHECKQ Q[3..0]= #H1

84     |

85     |;---    Shifting three positions to the right (same as one to the left),

86     |;       four times

87     |SETF  sel1 sel2

88     |FOR X:= 1 TO 4 DO

89     |        BEGIN

90     |             CLOCKF CLK

91     |        END

92     |CHECKQ Q[3..0]= #H1

93     |

94     |TRACE_OFF

95     |

96     |

97     |

98     |

%% MACHPAR %%    ERROR count:  1     WARNING count:  0

%% MACHPAR %%    File processing terminated.     File:  bar_4xx.pds

Other program modules provide descriptive error messages that help identify the cause of the error condition.

Warning messages explain actions taken by the MACHXL software to resolve ambiguities in the design file.  For example, the MACHXL software generates a warning message if the design file contains more than one equation for a given pin or node (in which case the equations are ORed).

Whenever possible, the MACHXL Fitter implements clocks as global clocks and macrocells as synchronous macrocells.  The MACHXL Fitter generates warning messages when it is obliged, due to resource constraints, to implement ambiguous clock signals (those that are neither forced global nor forced non-global) as product-term clocks, as shown in the log file fragment below.

MACHXL  MACHFITR

COPYRIGHT (c) ADVANCED MICRO DEVICES INC., 1993

*** Source file is Discard.pds . Device is MACH465 .

|> WARNING z5112 - Signals CLK1_USER ( node ) and IPIN1 are input paired

but the node is clocked by a product term clock CLOCK1 .

The input pairing is discarded.

|> WARNING z5112 - Signals CLK2_USER1 ( node ) and IPIN21 are input paired

but the node is clocked by a product term clock CLOCK2 .

The input pairing is discarded.

|> WARNING z5112 - Signals CLK2_USER2 ( node ) and IPIN22 are input paired

but the node is clocked by a product term clock CLOCK2 .

The input pairing is discarded.

|> INFO z5065 - For outputs, implicit output enables will be set to VCC.

|> INFO z5070 - Implicit set/reset equations will be set to DONT_CARE.

Some clock signals that are eligible to be global clocks are instead implemented as non-global clocks due to resource constraints, as shown in the log file fragment below.

*** End of Pla2db.

Check preplaced pins/nodes

Check preplaced blocks

Check unreferenced pins/nodes

Check clock rules

List of global clocks:

CLOCK3:

...... Controls a floating input register/latch.

CLOCK4:

...... Controls a floating input register/latch.

CLOCK5:

...... Controls a floating input register/latch.

SPECIAL_CLOCK:

...... Controls a signal with both SET & RESET non-GND.

Continued...

...Continued

List of non-global clocks:

CLOCK1:

...... Global clock capacity exceeded.

CLOCK2:

...... Global clock capacity exceeded.

*** End of Normalization.

*** End of DRC.

*** Partitioning successful.

*** Routing successful. Assembler invoked.

|> INFO z5088 - Single-literal clock signal used as product term clock in

block C. Clock is CLOCK1 (Pin 31).

|> INFO z5088 - Single-literal clock signal used as product term clock in

block D. Clock is CLOCK2 (Pin 22).

...... Zero Hold Time For Input Registers? N

*** The JEDEC file generated is Discard.jed .

*** Report Generator invoked.

Partitioning 100% - Completed

Placement    100% - Completed

Routing      100% - Completed

%%% Fitting process is successful %%%

*** Report Generator end.

%%%% MACHFITR %%%% Fitting successful. File Discard.pds .

%%%% MACHFITR %%%% ERROR count 0 WARNING count 3 .

Use the information in the list of global clocks, the list of non-global clocks, and the "INFO z5088" messages to determine if signals you intended to be global clocks were delivered to a macrocell as a product-term clock rather than through the block clock mechanism.  Refer to "Understanding Global Clocks" in Chapter 8, "Using the Fitter," for a detailed discussion of clocks, including rules governing how the Fitter chooses signals to be implemented as global or non-global clocks.  "Manually Forcing a Clock Signal to be Global" in Chapter 8, "Using the Fitter," explains how to force the Fitter to implement a signal as a global clock.

# Fitting Report

The fitting report provides summary information for the user to analyze either success or failure of the current fitting pass.  In the event of a failure to partition, place, or route, the fitting report gives a different set of information indicating why the design is not realizable with respect to the device architecture resources and/or fitting options.

The fitting report contains the full information if the fitting pass is successful, partial information if the partitioning, placement or routing are incomplete.  Data fields for which data is not available are indicated with an ellipsis (...).

## Header Information

The first section of the Fitting Report contains the name of the design file and the date and time the Fitter started to process the design.

MACHXL  MACHFITR

COPYRIGHT (c) ADVANCED MICRO DEVICES INC., 1993

******************************************************************

* Design Name = test.pds, Device = MACH435, Oct 20 10:11:33 1993 *

******************************************************************

## MACH Fitter Options

This section of the fitting report contains a list of options used in the current fitting pass.  Each option is presented as it appears in the menu.  Only options that affect fitting results are contained in the options list.

The sample "MACH Fitter Options" report fragment below contains default settings for each family of devices.

MACH 1xx/2xx

***********************

* MACH FITTER OPTIONS *

***********************

SIGNAL PLACEMENT:

Handling of Preplacements                      No Change

Use placement data from                        Design file

FITTING OPTIONS:

SET/RESET treated as DONT_CARE?                N

Iterate between partitioning and place/route?  Y

Balanced partitioning?                         N

Spread Placement?                              Y

Maximun Run Time                               15 minutes

MACH 3xx/4xx

***********************

* MACH FITTER OPTIONS *

***********************

SIGNAL PLACEMENT:

Handling of Preplacements                      No Change

Use placement data from                        Design file

FITTING OPTIONS:

Global clocks routable as PT clocks?           N

22V10/MACH1XX/2XX S/R Compatibility?           Y

SET/RESET treated as DONT_CARE?                N

Reduce Unforced Global Clocks?                 N

Iterate between partitioning and place/route?  Y

Balanced partitioning?                         N

Reduce Routes Per Placement?                   N

Maximun Run Time                               15 minutes

Note that options not available for MACH 1xx/2xx are not listed.

## Device Resource Summary

The Device Resource Summary table displays device-level pin, macrocell, and product term utilization and shows which resources might be available for logic additions and changes.

The "Input Registers" field is not shown for devices that do not have dedicated input registers.  Additionally, the "> 1 PT Macrocells" and "1 PT Macrocells"  fields are not shown for devices that do not have an XOR product term.

MACH435 (device has dedicated input registers and XOR product term)

***************************

* DEVICE RESOURCE SUMMARY *

***************************

Total

Available  Used  Available  Utilization

Dedicated Pins

Input-Only Pins                 2        0      2    -->     0%

Clock/Input Pins                4        1      3    -->    25%

I/O Pins                         64       12     52    -->    18%

Input Registers                  64        0     64    -->     0%

Central Switch Matrix Outputs   264       12    252    -->     4%

Product Term Clusters           128        4    124    -->     3%

Logical Product Terms           640       16    624    -->     2%

Logic Macrocells                128        4    124    -->     3%

> 1 PT Macrocells              ..        4    124

1 PT Macrocells                ..        0      0

Unusable Macrocells            ..        0     ..

MACH111 (device has neither dedicated input registers nor XOR product term)

***************************

* DEVICE RESOURCE SUMMARY *

***************************

Total

Available  Used  Available  Utilization

Dedicated Pins

Input-Only Pins                 2        1      1    -->    50%

Clock/Input Pins                4        3      1    -->    75%

I/O Pins                         32        9     23    -->    28%

Central Switch Matrix Outputs    52       12     40    -->    23%

Product Term Clusters            32        4     28    -->    12%

Logical Product Terms           128       16    112    -->    12%

Logic Macrocells                 32        4     28    -->    12%

Input Registers                ..       ..     ..

Unusable Macrocells            ..        0     ..

The labels and abbreviations used in the "Device Resources" section of the fitting report are described below.

| Dedicated Pins | Reports the number and utilization of dedicated input-only pins and clock/input pins in the device. |
| --- | --- |
| I/O Pins | Reports the number and utilization of I/O pins in the device, whether used for input, output, or both. |
| Input Registers (Devices with dedicated input registers) | Reports the number and utilization of dedicated input registers in the device. |
| Central Switch Matrix Outputs | Reports the number and utilization of outputs from the central switch matrix. |
| Product Term Clusters | Reports the number and utilization of product term clusters in the device. |
| Logical Product Terms | Reports the number and utilization of product terms in the device's sum-of-products arrays. |
| Logic Macrocells | Reports the number of and utilization of logic macrocells in the device. |
| >1 PT Macrocells (Devices with XOR PT) | Reports the number and utilization of macrocells having more than one available logic product term. (Not available if partitioning fails.) |
| 1 PT Macrocells (Devices with XOR PT) | Reports the number and utilization of macrocells that have only one product term (the XOR product term) available. (Not available if partitioning fails.) |
| Unusable Macrocells | Reports the number of macrocells that are neither >1PT nor 1PT. (Not available if partitioning fails.) |

# Block Partitioning Summary

The block partition summary table gives a summary of resources used by each block.

For devices without dedicated input registers, the "Inp Reg" field is deleted and a single number is given in the "Macrocells available" column since these devices do not have the option for "1 PT" and ">1 PT" (see example below).

MACH435 (device has dedicated input registers)

******************************

* BLOCK PARTITIONING SUMMARY *

******************************

Macrocells  # of PT

Logic   I/O    Inp    Macrocells    available   clusters

Fanin    PTs    Pins   Reg   Used Unusable  1PT   >1PT  available

|---------------------------------------------------------------------------|

Maximum     33     80      8      8     ..     ..         16        16

|---------------------------------------------------------------------------|

Block A     12     16      8      0      4      0      0     12     12

Block B      0      0      4      0      0      0      0     16     16

Block C      0      0      0      0      0      0      0     16     16

Block D      0      0      0      0      0      0      0     16     16

Block E      0      0      0      0      0      0      0     16     16

Block F      0      0      0      0      0      0      0     16     16

Block G      0      0      0      0      0      0      0     16     16

Block H      0      0      0      0      0      0      0     16     16

> Four rightmost columns above reflect last status of the placement process.

MACH111 (device has neither dedicated input registers nor XOR product term)

******************************

* BLOCK PARTITIONING SUMMARY *

******************************

# of PT

Logic   I/O           Macrocells    Macrocells  clusters

Fanin    PTs    Pins         Used Unusable  available   available

|---------------------------------------------------------------------------|

Maximum     26     64     16            ..     ..         16        16

|---------------------------------------------------------------------------|

Block A     12     16      9             4      0         12        12

Block B      0      0      0             0      0         16        16

> Two rightmost columns above reflect last status of the placement process.

| ã | Note: When partitioning fails, the partitioning module returns information extracted from the last partition acquired. The information from this table can be used to determine the distribution of objects between blocks. When partitioning fails, the Block Partitioning Summary columns "Macrocells Unusable," Macrocells >1PT," and "Macrocells 1PT," are not listed. |
| --- | --- |

The labels and abbreviations used in the "Block Partitioning Summary" section of the fitting report are described below.

| Fanin | Number of distinct signals that fanin from the central switch matrix into the block. |
| --- | --- |
| Logic PTS | Number of product terms used by data terms in logic equations placed in the block. (Does not include clock or Set/Reset product terms associated with asynchronous macrocells.) |
| I/O Pins | Number of I/O pins used for input, output or both in the block. |
| Inp Reg | Number of input registers used in the block. |
| Macrocells used | Number of macrocells used in the block. |
| Macrocells unusable | Number of macrocells in block that are not used and cannot be used with the current placement. (This column does not appear if the design fails to fit.) |
| Macrocells available 1 PT | Number of macrocells in the block that have only one product term. |
| Macrocells available >1 PT | Number of macrocells in the block that have more than one product term. |
| # of PT clusters available | Remaining number of product-term clusters usable by signals partitioned to the block. |

## Signal Summary

This section of the fitting report provides a detailed representation of Fitter placements.  It  can be used to determine block fanout and fanin information.

MACH 3xx/4xx

******************

* SIGNAL SUMMARY *

******************

Pin/Node Logic

Signals Block Loc PTs XOR   Type   Type    Fanout

|---------------------------------------------------------------------------|

DATA[0]   B    15  .   .    input   ...   A-------

DATA[1]   B    14  .   .    input   ...   A-------

DATA[2]   B    13  .   .    input   ...   A-------

DATA[3]   B    12  .   .    input   ...   A-------

ENA   A     3  .   .    input   ...   A-------

Q[0]   A     6  4   .  i/o pin   ...   --------

Q[1]   A     4  4   .  i/o pin   ...   --------

Q[2]   A    10  4   .  i/o pin   ...   --------

Q[3]   A     8  4   .  i/o pin   ...   --------

RESET   A     5  .   .    input   ...   A-------

SEL1   A     9  .   .    input   ...   A-------

SEL2   A     7  .   .    input   ...   A-------

RN_Q[0]   A   A12  4   G  implied   D/S   A-------

RN_Q[1]   A    A8  4   G  implied   D/S   A-------

RN_Q[2]   A    A4  4   G  implied   D/S   A-------

RN_Q[3]   A    A0  4   G  implied   D/S   A-------

| ã | Note: For MACH 1xx/2xx devices, XOR information is not applicable and will be deleted. |
| --- | --- |

The labels and abbreviations used in the "Signal Summary" section of the fitting report are described below.

| Signals | Name of the signal. A node name that is the same as a pin name indicates that the node was created by the Fitter to provide a register or latch for a pin declared as REGISTERED or LATCHED, for which no corresponding node was declared in the design. By contrast, a node name than is the same as the pin name but preceded by "RN_" (for "referenced node") indicates that the node was created by the Fitter to provide a register or latch for a pin declared as REGISTERED or LATCHED, for which no corresponding node was declared in the design, and that the register/latch feedback was referenced in the design by another signal. |
| --- | --- |
| Pin/Node | "Pin" if the signal corresponds to a pin; "Node" if the signal corresponds to a node. |
| Block | Letter corresponding to the PAL block in which the signal is placed. |
| Loc | Tells where the signal was placed. If placed at a pin, this field contains the physical pin number. If placed at a macrocell, this field contains the node designation (consisting of a block letter and a node number) of the macrocell. If placed at an input register, this field has the form Xir-Input node number where X is a block designator. |
|  | Example Bir-2 ß indicates input node 2 in block B (Refer to the pin and node summary for the target MACH device in Chapter 10, "Device Reference," for a list of node numbers. ) |
| PTs | The number of product terms connected to the signal. |
| XOR | "G" if the XOR is unused; "P" if the XOR is used as a product term or for polarity control. |
| Type | Type of signal: buried, input, output, I/O pin, opair (output macrocell paired with an output pin), ipair (input macrocell paired with an input pin), or implied. "Implied" indicates a node that was created by the Fitter and either a) named (because it was referenced as register feedback by another signal) or b) unnamed (because it was created to provide a register or latch for a pin declared as REGISTERED or LATCHED but never referenced as register feedback). |
| Fanout | Block(s) to which the logic signal is routed. |

## PRESET, RESET and OUTPUT ENABLED Signal SummaryXE "PRESET, RESET, and OUTPUT ENABLED signal summary"§XE "reports:PRESET, RESET, and OUTPUT ENABLED signal summary"§

(MACH 1xx/2xx devices only)

This section summarizes product term driven PRESET, RESET and OUTPUT ENABLE signals of the design. It shows how these functional signals are used in various PAL blocks and can be used as a reference to swap members between blocks.

**************************************************

* PRESET, RESET and OUTPUT ENABLE signal SUMMARY *

**************************************************

PRESET signal summary:

Number      Pin or Node Name      Block

|--------------------------------------------------|

RESET signal summary:

Number      Pin or Node Name      Block

|--------------------------------------------------|

1                  Q[0]          A-

OUTPUT ENABLE signal summary:

Number      Pin or Node Name      Block

|--------------------------------------------------|

1                  Q[0]          A-

All PRESET signals that have the same boolean equation in the design are represented by a unique number in the PRESET Signal Summary table. A selected name in the group is represented in "Pin or Node name" field. The "Block" field records all the blocks that have the same preset signal. RESET and OUTPUT ENABLE signals work the same way.

## Tabular Information

This section of the fitting report can be used to diagnose incomplete partitions, incomplete placements and incomplete routing.

MACH215 and MACH 3xx/4xx

***********************

* TABULAR INFORMATION *

***********************

DEDICATED PINS

Logic             Clock

Pin              Signal        Type          Fanout            Fanout

|------------------------------------------------------------------------|

20                 CLK     clk/inp          --------          A-------

23      ..............          ..          ........          ........

41      ..............          ..          ........          ........

62      ..............          ..          ........          ........

65      ..............          ..          ........          ........

83      ..............          ..          ........          ........

**********************************

* Signals - Equations Where Used *

**********************************

Signal Source :    Fanout List

|---------------------------------------------------------------------------|

DATA[0]:           Q[0]{A}

{A}

DATA[1]:           Q[1]{A}

{A}

DATA[2]:           Q[2]{A}

{A}

DATA[3]:           Q[3]{A}

{A}

RN_Q[0]:           Q[1]{A}           Q[2]{A}           Q[3]{A}

{AAA}

RN_Q[1]:           Q[0]{A}           Q[2]{A}           Q[3]{A}

{AAA}

RN_Q[2]:           Q[0]{A}           Q[1]{A}           Q[3]{A}

{AAA}

RN_Q[3]:           Q[0]{A}           Q[1]{A}           Q[2]{A}

{AAA}

SEL1:           Q[0]{A}           Q[1]{A}           Q[2]{A}

:           Q[3]{A}

{AAA A}

Continued...

...Continued

SEL2:           Q[0]{A}           Q[1]{A}           Q[2]{A}

:           Q[3]{A}

{AAA A}

RESET:           Q[0]{A}           Q[1]{A}           Q[2]{A}

:           Q[3]{A}

{AAA A}

ENA:           Q[0]{A}           Q[1]{A}           Q[2]{A}

:           Q[3]{A}

{AAA A}

Block A singular list (Input drives only one logic equation)

DATA[0]:           Q[0]	;Compare with "Block A singular list"

DATA[1]:           Q[1]	;section for MACH 1xx/2xx as shown in

DATA[2]:           Q[2]	;the report sample that follows

DATA[3]:           Q[3]	;this one.

BLOCK A CLOCK MUX

Block Clocks     Signal    Pin   Pol	;This section does not appear in any

|-----------------------------------|	;MACH 1xx/2xx report files

Ck0                 CLK     20    H

Ck1                 ...     ..    .

Ck2                 ...     ..    .

Ck3                 ...     ..    .

;Note how this section differs from

BLOCK A LOGIC MACROCELLS & INPUT REGISTERS	;the "BLOCK A LOGIC MACROCELLS"

PTS 	;section in the MACH 1xx/2xx report

T   U   A	;sample that follows.

L                            Y   S   V  X  P   C  S  R   P

O                            P   E   A  O  O   L  E  E   I

C     Node          Signal   E   D   L  R  L   K  T  S   N   Fanout

|---------------------------------------------------------------------------|

A0      2          RN_Q[3]  D/S  4  11  G  H Ck0  G B0   8  A-------

A1      3 ................  ...  .  15  .  .   .  .  .  ..  ........

A2      4 ................  ...  .  15  .  .   .  .  .  ..  ........

A3      5 ................  ...  .  15  .  .   .  .  .  ..  ........

A4      6          RN_Q[2]  D/S  4  16  G  H Ck0  G B0  10  A-------

A5      7 ................  ...  .  15  .  .   .  .  .  ..  ........

A6      8 ................  ...  .  15  .  .   .  .  .  ..  ........

A7      9 ................  ...  .  15  .  .   .  .  .  ..  ........

A8     10          RN_Q[1]  D/S  4  16  G  H Ck0  G B0   4  A-------

A9     11 ................  ...  .  15  .  .   .  .  .  ..  ........

A10    12 ................  ...  .  15  .  .   .  .  .  ..  ........

A11    13 ................  ...  .  15  .  .   .  .  .  ..  ........

A12    14          RN_Q[0]  D/S  4  16  G  H Ck0  G B0   6  A-------

A13    15 ................  ...  .  15  .  .   .  .  .  ..  ........

Continued...

...Continued

A14    16 ................  ...  .  15  .  .   .  .  .  ..  ........

A15    17 ................  ...  .  10  .  .   .  .  .  ..  ........

Air-0 130 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-1 131 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-2 132 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-3 133 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-4 134 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-5 135 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-6 136 ................  ...  .   .  .  .   .  .  .  ..  ........

Air-7 137 ................  ...  .   .  .  .   .  .  .  ..  ........

;Note differences from the "BLOCK A I/O Pins" section

BLOCK A I/O PINS	;in the MACH 1xx/2xx report sample that follows this one.

Pin onode inode           Signal     Type    Fanout

|----------------------------------------------------------------|

3  ...   ...               ENA     input  A-------

4   10   ...              Q[1]   i/o pin  --------

5  ...   ...             RESET     input  A-------

6   14   ...              Q[0]   i/o pin  --------

7  ...   ...              SEL2     input  A-------

8    2   ...              Q[3]   i/o pin  --------

9  ...   ...              SEL1     input  A-------

10    6   ...              Q[2]   i/o pin  --------

BLOCK A LOGIC ARRAY FANIN

CSM             Signal    Source        CSM             Signal    Source

|----------------------------------|    |----------------------------------|

mxA0              ...           ...     mxA17            SEL2         pin 7

mxA1          DATA[2]        pin 13     mxA18         RN_Q[1]     mcell A-8

mxA2          DATA[3]        pin 12     mxA19         RN_Q[2]     mcell A-4

mxA3            RESET         pin 5     mxA20             ...           ...

mxA4          DATA[0]        pin 15     mxA21             ...           ...

mxA5              ...           ...     mxA22             ...           ...

mxA6             SEL1         pin 9     mxA23             ...           ...

mxA7              ...           ...     mxA24             ...           ...

mxA8              ENA         pin 3     mxA25             ...           ...

mxA9          DATA[1]        pin 14     mxA26             ...           ...

mxA10             ...           ...     mxA27             ...           ...

mxA11             ...           ...     mxA28             ...           ...

mxA12             ...           ...     mxA29         RN_Q[3]     mcell A-0

mxA13             ...           ...     mxA30             ...           ...

mxA14             ...           ...     mxA31             ...           ...

mxA15         RN_Q[0]    mcell A-12     mxA32             ...           ...

mxA16             ...           ...

Continued...

...Continued

BLOCK B I/O PINS

Pin onode inode           Signal     Type    Fanout

|----------------------------------------------------------------|

12  ...   ...           DATA[3]     input  A-------

13  ...   ...           DATA[2]     input  A-------

14  ...   ...           DATA[1]     input  A-------

15  ...   ...           DATA[0]     input  A-------

16  ...   ...  ................        ..  ........

17  ...   ...  ................        ..  ........

18  ...   ...  ................        ..  ........

19  ...   ...  ................        ..  ........

; similar information listed for other blocks used, if any

MACH435 report file key:

A             - Asynchronous mode

AVAL          - Additional product terms available within the current

steering allocation, plus those potentially available

through resteering of free clusters.

B0            - Block Asynchronous Reset/Preset product term 0

B1            - Block Asynchronous Reset/Preset product term 1

C             - Combinatorial

Ck0           - Block clock generated from pin 20 or pin 23

Ck1           - Block clock generated from pin 20 or pin 23

Ck2           - Block clock generated from pin 62 or pin 65

Ck3           - Block clock generated from pin 62 or pin 65

clk           - Clock

CSM           - Central Switch Matrix

D             - D-type flip flop

G             - Ground

H             - High

implied       - Node occupying the macrocell drives the output pin

but not defined in the design file.

inode         - Input node

Inp           - Input

ipair         - Input paired node

I/O           - Input or Output

L             - Low

L             - Latch

LOC           - Location

mcell <X>     - Source is macrocell from block <X>

Mux           - Multiplexer

mx            - Block Array input multiplexer

onode         - Output node

Continued...

...Continued

opair         - Output paired node

P             - Product Term

Pol           - Polarity

PT(s)         - Product term(s)

Reg           - Register

Res           - Reset control

RN_<pin_name> - Output node paired with <pin_name> created by Fitter.

S             - Synchronous mode

Set           - Preset control

T             - T-type flip flop

XOR           - Exclusive OR gate

<X>ir         - Input register in block <X>

.             - Not available or Not applicable

Partitioning 100% - Completed

Placement    100% - Completed

Routing      100% - Completed

%%% Fitting process is successful %%%

MACH 1xx/2xx (except MACH215)

Block A singular list (Input drives only one logic equation)

DATA[0]:           Q[0]

DATA[1]:           Q[1]

DATA[2]:           Q[2]

DATA[3]:           Q[3]

BLOCK A LOGIC MACROCELLS

PTS

T   U   A

L                            Y   S   V  P   C  S  R   P

O                            P   E   A  O   L  E  E   I

C     Node          Signal   E   D   L  L   K  T  S   N   Fanout

|---------------------------------------------------------------------------|

A0      2          RN_Q[0]   D   4   4  H Ck3  .  1   2  A-

A1      3 ................  ...  .   8  .   .  .  .  ..  ..

A2      4 ................  ...  .   8  .   .  .  .  ..  ..

A3      5          RN_Q[1]   D   4   8  H Ck3  .  1   5  A-

A4      6 ................  ...  .   8  .   .  .  .  ..  ..

A5      7 ................  ...  .   8  .   .  .  .  ..  ..

A6      8          RN_Q[2]   D   4   8  H Ck3  .  1   8  A-

A7      9 ................  ...  .   4  .   .  .  .  ..  ..

A8     10 ................  ...  .   4  .   .  .  .  ..  ..

A9     11          RN_Q[3]   D   4   8  H Ck3  .  1  15  A-

Continued...

...Continued

A10    12 ................  ...  .   8  .   .  .  .  ..  ..

A11    13 ................  ...  .  12  .   .  .  .  ..  ..

A12    14 ................  ...  .  12  .   .  .  .  ..  ..

A13    15 ................  ...  .  12  .   .  .  .  ..  ..

A14    16 ................  ...  .  12  .   .  .  .  ..  ..

A15    17 ................  ...  .   8  .   .  .  .  ..  ..

BLOCK A I/O PINS

Output

Pin onode        Enable           Signal     Type    Fanout

|----------------------------------------------------------------|

2    2            1               Q[0]   i/o pin  --

3  ...          ...                ENA     input  A-

4  ...          ...              RESET     input  A-

5    5            1               Q[1]   i/o pin  --

6  ...          ...               SEL2     input  A-

7  ...          ...               SEL1     input  A-

8    8            1               Q[2]   i/o pin  --

9  ...          ...            DATA[3]     input  A-

14  ...          ...   ................        ..  ..

15   11            1               Q[3]   i/o pin  --

16  ...          ...   ................        ..  ..

17  ...          ...   ................        ..  ..

18  ...          ...   ................        ..  ..

19  ...          ...   ................        ..  ..

20  ...          ...   ................        ..  ..

21  ...          ...   ................        ..  ..

The information from the tabular form shown above will be incomplete in cases where partitioning, placing, routing fail.  Pin and node numbers are not available on failure to partition.  Signal names partitioned on the last attempt of the partitioner will appear in the table.  Data fields for which data is not available are indicated with an ellipsis (...).

The labels and abbreviations used in the "Tabular Information" section of the fitting report are described below.

| DEDICATED PINS |  | For MACH architecture these are pins that are dedicated inputs and or clocks. |  |
| --- | --- | --- | --- |
|  | Pin |  | Physical pin number. |
|  | Signal |  | String of up to 14 characters representing the signal name. |
|  | Type |  | Dedicated types are "clk/inp," "input," or (MACH465 only) "Clock." |
|  | Logic Fanout |  | Indicates to which block the logic signal fans out as logic. |
|  | Clock Fanout |  | Indicates to which block the clock signal fans out through the block clock mechanism. |
| Signals - Equations Where Used |  | For each signal s, lists all signals that reference signal s. Below the list of equations that reference signal s is a list (enclosed in braces) of the PAL blocks that contain each of the referencing signals. PAL block letters are listed in the same order in which the signals to which they correspond are listed. |  |
| BLOCK-x CLOCKS MUX (MACH 3xx/4xx devices only) |  | (Where x denotes the letter that corresponds to the PAL block.) |  |
|  | Block Clocks |  | Clock mux outputs "CK0".."CK3." |
|  | Signal |  | String of up to 14 characters representing the signal name. |
|  | Pin |  | Physical pin number. |
|  | Pol |  | Polarity "H" (active high) or "L" (active low). |
| BLOCK-x LOGIC MACROCELLS & INPUT REGISTERS |  | (Where x denotes the letter corresponding to the PAL block.) An exhaustive list of logic macrocells and input registers. (The legend "& INPUT REGISTERS" is omitted from reports for devices that have no dedicated input registers.) |  |
|  | LOC |  | Relative node number, in the block, for logic or input cell. Input cell numbers are preceded by a "<block>ir-." |
|  | Node |  | Software node number of macrocell. This is the same number as in the JEDEC Specification and the MACHXL language. |
|  | Signal |  | String of up to 14 characters representing the signal name. |
|  | TYPE |  | Synchronous/Asynchronous devices: Given in the form <register_type> <macrocell_type>, where register_type is "D," "T," "L" (latch) or "C" (combinatorial) and macrocell_type is "S" (synchronous), "A" (asynchronous), or "." (combinatorial). For example, "D/S" refers to a D-type flip-flop implemented in a synchronous macrocell. Synchronous devices: Given in the form <register_type>, where register_type is "D," "T," "L" (latch) or "C" (combinatorial). For example, "T" refers to a T-type flip-flop. |
|  | PTS USED |  | Number of product terms used by equation placed in this cell. |
|  | PTS AVAL |  | Additional product terms available within the current steering allocation, plus those potentially available through re-steering of free clusters. If the macrocell is not used, this field contains the maximum number of product terms (its own and those that can be steered to it). If the macrocell is used, this field contains the number of product terms available after all clusters are routed. |
|  | XOR (MACH 3xx/4xx devices only) |  | "G" means the XOR gate of the cluster aligned with the signal's macrocell is connected to ground. "P" means one of the XOR gates is connected to a product term or is used for polarity control. The symbol "." means the status is unknown. |
|  | POL |  | Signal polarity "H" (active high), "L" (active low), or "." if no signal placed at this location. Polarity is "H" if the polarity of PIN (NODE) statement matches that of equation, otherwise polarity is "L.". |
|  | CLK |  | Block clocks CK0..CK3 from clock mux (synchronous mode), "P" for product term (asynchronous mode), and "." for none. |
|  | SET |  | Set signal: "P" for product term, "G" for ground, "B0" for block set/reset ARP1, "B1" for block set/reset APR2, and "." for none. "B0" and "B1" occur only in synchronous modes. |
|  | RES |  | Reset signal: "P" for product term, "G" for ground, "B0" for block set/reset ARP1, "B1" for block set/reset ARP2, "." for none. "B0" and "B1" occur only in synchronous modes. |
|  | PIN |  | Indicates which pin (by number) is connected to this node. If the node is buried, the entry is "." |
|  | Fanout |  | Indicate to which block the node signal fans out. |
| BLOCK x I/O PINS |  | (Where x corresponds to the PAL block.) There is one table for each physical block. For MACH architecture this is a list of I/O pins in a block. |  |
|  | Pin |  | Physical pin number. |
|  | onode |  | Indicates output nodes connected to I/O pin. This is the software node number as in the JEDEC Specification and the MACHXL language. |
|  | inode (Not for MACH355 or MACH 1xx devices) |  | Indicates input nodes connected to I/O pin. This is the software node number as in the JEDEC Specification and the MACHXL language. |
|  | Output Enabled |  | The "OUTPUT ENABLE" field is added for MACH 1xx/2xx devices. Its value can be represented by V(Vcc), G(Gnd) or a product term. In the case of a product term, a number is shown in this column which is the entry in OUTPUT ENABLE signal summary table under "PRESET, RESET and OUTPUT ENABLE signal summary." |
|  | Signal |  | String of up to 14 characters representing the signal name. |
|  | Type |  | Type of pin "output", "input" or "i/o pin". |
|  | Fanout |  | Indicates to which block the pin signal fans out. |
| BLOCK-x LOGIC ARRAY FANIN |  | (Where x denotes the letter that corresponds to the PAL block.) This table contains routing information for each signal that fan in to the block. |  |
|  | CSM |  | Central Switch Matrix fanout to logic array of a PAL block. String of up to 14 characters representing the signal name. |
|  | Source |  | Given in the format <source_type> <location>, where source_type is the macrocell inode pin and location is the physical location. (Source drives physical.) |

## Fitting Status

This group of statements ends the fitting report.  In the case of a successful fitting pass (no errors generated), the partitioning, placement, and routing statements show 100% completion.

Partitioning 100% - Completed

Placement    100% - Completed

Routing      100% - Completed

%%%Fitting process is successful%%%

| ã | Note: Compilation can fail even if all three status fields (Partitioning, Placement, and Routing) show 100% completion, if the assembler fails to assemble the design. |
| --- | --- |

In the case of an unsuccessful fitting pass, the last successful module (partitioning or placement) shows 100% completion, the unsuccessful module shows a completion of less than 100%, and any subsequent module shows 0% completion.  Refer to "Failure Reports," later in this chapter, for more information.

# Place and Route Data Report

A Place and Route Data (Design.PRD) file is generated by the MACHXL Fitter when partitioning is successful. This file contains detailed tables and listings showing:

1.	Place and route processing time

2.	Place/Route Resource and Usage tables

3.	Signal fan-out table sorted in alphabetical order

4.	Device pin-out list

5.	Block information

a.	Logic block macrocell assignments and PT cluster steering

b.	Maximum PT capacity per block

c.	Node to I/O Pin mappings via the output switch matrix

d.	I/O Pin-to-node mappings

e.	I/O Pin-to-node and I/O Pin-to-input register pairings

6.	Input and Central switch matrix tables

a.	Input Multiplexer assignments

b.	Logic block fan-in through Central Switch Matrix

| ã | Note: No information is reported for blocks that are unused. Note: Some tables in the PRD file may extend beyond 78 columns. If you cannot view all of the report columns using the viewer accessed from the MACHXL View menu, use the text editor to examine the file instead. If the MACHXL Fitter successfully placed and routed the design, then the designer can use the information in these tables and listings to determine where more logic product terms or input signals can be placed. |
| --- | --- |

## Unplaceable Designs

If the MACHXL Fitter cannot place all the signals in the design, then the PRD file will show the best placement the Fitter found and will also list the unplaced signals. The designer can then verify that the signal cannot fit into the specified block, and can then study the macrocell assignments in the other logic blocks to determine if there is space in the other blocks to which the unplaceable signals can be moved.

## Unroutable Designs

The Placer will attempt to move signals around to use different routing resources, but will be constrained by preplacements.

The PRD file will mark the signals and the blocks to which they cannot route. The designer can reduce the amount of logic in the design to release more routing resources for the remaining signals, or can increase the amount of processing time allocated for the Fitter if it timed out.

### Place and route processing time

This section shows the start and duration of the place and route process.

For example:

Start: Fri Oct 07 10:06:17 1994

End  : Fri Oct 07 10:06:18 1994    $$$ Elapsed time: 00:00:01

===========================================================================

C:\MACHXL\DAT\MACH435 Design [Bar_4xx.pds]

### Place/Route Resource and Usage tables

The place/route resource usage table shows total available resources, how many were required and how many were successfully allocated and used.

The sample placement completion section below shows the number of macrocells and I/O pins available per block (that is, 16 macrocells and 8 I/O pins in block A), how many signals were assigned to the blocks (that is, 8 logic signals or signals requiring product terms partitioned to block A, with 6 I/O pins required), and how many were actually used. This means you can put up to 8 more logic signals, if product term clusters are still available, and 2 more inputs through the unused I/O pins in block A.

* Placement Completion

+- Block                       +------- IO Pins Available

|    +- Macrocells Available   |    +-- IO Pins Used

|    |    +- Signals to Place  |    |             +----- Logic Array Inputs

|    |    |    +- Placed       |    |             |   +- Array Inputs Used

_|____|____|____|_______________|____|_____________|___|________________

A | 16 |  8 |  8 => 100% |     8 |  6 =>  75% |  33 | 18 =>  54%

B | 16 |  7 |  7 => 100% |     8 |  7 =>  87% |  33 | 18 =>  54%

C | 16 |  9 |  9 => 100% |     8 |  8 => 100% |  33 | 27 =>  81%

D | 16 |  8 |  8 => 100% |     8 |  7 =>  87% |  33 | 26 =>  78%

E | 16 |  5 |  5 => 100% |     8 |  8 => 100% |  33 | 16 =>  48%

F | 16 |  4 |  4 => 100% |     8 |  7 =>  87% |  33 | 22 =>  66%

G | 16 |  7 |  7 => 100% |     8 |  8 => 100% |  33 | 16 =>  48%

H | 16 |  5 |  5 => 100% |     8 |  8 => 100% |  33 | 17 =>  51%

---|----|----|------------|-------|------------|-----|------------------

The logic array input figure indicates the maximum number of signals that can be routed into a block, and how many are used. In the preceding example, 33 signals can be routed into a MACH435 block, and the design uses 18 of the block inputs for the 8 logic signals placed in block A. This means that up to 15 more signals can be routed into block A, but this is dependent on their being routed through the switch matrix.

The Input/Clock signal count figure indicates the number of input or clock signals in the design, and how many were successfully assigned to either I/O, dedicated input, or clock/input pins.

* Input/Clock Signal count:  24 -> placed:  24 = 100%

The next table shows the number of dedicated input and clock/input pins in the device, and how many were used.

Available   Used

-----------------------------------------------------------------

Input Pins            :   2        2    => 100%

Clock, Clk/Input Pins :   4        4    => 100%

The routing completion display measures how many signals have been routed, as a percentage of the total number of signals to be routed. If a signal is not routed, then the signal in the fan-out table (next section) will have a '~' in front of the block that it could not route to.

The number of placement and routing attempts (counting from 0) is shown under the routing completion data.

* Routing Completion: 100%

* Attempts: Place [     265] Route [       0]

Example
If routing was successful on first routing attempt of the 265th placement attempt, you would see Place[264] Route [0], because placing and routing attempts are numbered from 0.

### Signal Fan-Out Table

The fan-out table contains a list of signals in the design, sorted in alphabetical order, and the blocks that each signal fans-out to. A sample fan-out table is as follows:

===========================================================================

Signal Fanout Table

===========================================================================

+- Signal Number

|                +- Block Location ('+' for dedicated inputs)

|                |  +- Sig Type

|                |  |    +- Signal-to-Pin Assignment

|  Signal Name   |  |    |           Fanout to Logic Blocks

__|________________|__|____|_______________________________________________

1|_16byte       |G|INP|  70|=> . . . D . F . .

2|_8byte        |H|INP|  78|=> . . . . . F G .

3|buswon        |A|NOD| N/A|=> A B C . E . . .

4|clk40         |+|Cin|  65|=> . . . . . . . .

...

58|rn_t4_s[0]    |D|NOD| N/A|=> A . . D . . . . => Paired w/: [       t4_s[0]]

59|rn_t4_s[1]    |D|NOD| N/A|=> A . . D . . . . => Paired w/: [       t4_s[1]]

...

95|t4_s10        |A|NOD| N/A|=> . . . . . F . .

96|t4_s[0]       |D| IO|  40|=> . . . . . . . . => Paired w/: [    rn_t4_s[0]]

97|t4_s[1]       |D| IO|  36|=> . . . . . . . . => Paired w/: [    rn_t4_s[1]]

98|t4_s[2]       |D| IO|  39|=> . . . . . . . . => Paired w/: [    rn_t4_s[2]]

...

---------------------------------------------------------------------------

The signals are printed in alphabetical order, followed by the block the signals are assigned to. If the signal is assigned to a dedicated input or clock pin, then this field is marked with a '+'. This is followed by the signal type, which can be one of the following:

| Cin | Clock/Input signal |
| --- | --- |
| CLK | Clock only signal |
| INP | Input signal |
| NOD | Internal/buried signal node |
| IO | IO signal |
| OUT | Output signal (no feedback) |
| Rin | Registered input signal |

This is followed by the blocks that the signals are assigned to. In the preceding example, internal node signal BUSWON fans-out to blocks A, B, C, and E. This means that BUSWON requires one block array input from each of these blocks.

Some signals may not have fan-out entries, such as OUTPUT signals, or IO signals that are fed back through the nodes that they are paired with. In the preceding example, IO signal T4_S[1] does not have feedback, but it is paired with the node RN_T4_S[1], which fans out to blocks A and D.

### Device pin-out list

This list shows the device pin type, and whether a signal has been fixed or preplaced to that pin or not.

===========================================================================

< C:\MACHXL\DAT\MACH435 Device Pin Assignments >

===========================================================================

===========================================================================

< Device Pin Assignments >

===========================================================================

+- Device Pin No

|     +- Pin Type

|     |          Signal Fixed (*)-+

|     |           Signal Name     |

____|_____|___________________________|____________________________________

1 |  GND |   |                -  | |

2 |  Vcc |   |                -  | |

3 |  I_O |A00|            nadl_o |*|   <-- signal is fixed

4 |  I_O |A01|          nsdstr_o |*|

5 |  I_O |A02|           nactroe |*|

6 |  I_O |A03|                -  | |   <-- Block A IO pin not used

7 |  I_O |A04|           sdstr_o |*|

8 |  I_O |A05|           t4_s[3] |*|

9 |  I_O |A06|                -  | |

10 |  I_O |A07|            ncmd_o |*|

11 |  GND |   |                -  | |

...

33 |  I_O |D07|                -  | |

34 |  I_O |D06|            sel_8b |*|

35 |  I_O |D05|        nodd_cycle |*|

36 |  I_O |D04|           t4_s[1] |*|

37 |  I_O |D03|           t2_s[1] |*|

38 |  I_O |D02|           t1_s[0] |*|

Continued...

...Continued

39 |  I_O |D01|           t4_s[2] |*|

40 |  I_O |D00|           t4_s[0] |*|

41 |  Inp |   |           en_xfer |*|

42 |  Vcc |   |                -  | |

...

65 | CkIn |   |             clk40 |*|

66 |  I_O |G00|           t3_s[0] |*|

67 |  I_O |G01|           t3_s[1] |*|

...

82 |  I_O |H00|          error[0] |*|

83 |  Inp |   |        stream_pos |*|

84 |  Vcc |   |                -  | |

---------------------------------------------------------------------------

### Block information

Each MACH block has macrocells, I/O pins, and block array inputs associated with it. The following sections will describe how these resources are used. Information on these resources is printed only if they are used.

#### Macrocell (MCell) Cluster Assignments

This section shows how the macrocells in a block and their associated product term clusters are allocated. A MACH435 macrocell can steer 5 product terms (4 PT and 1 XOR used as logic) from its own macrocell, the preceding macrocell and the succeeding 2 macrocells. Macrocell 1 can therefore use its own PT cluster, and can also steer the PT clusters from macrocells 0, 2, and 3 for a maximum of 20 product terms. If the designer assigns (that is, preplaces) a 20 PT signal to macrocell 0 in a block, then this placement is not realizable in a MACH435 because macrocell 0 does not have a preceding macrocell.

Example

The following macrocell cluster assignment listing shows synchronous output signal SDSTR_O which has 17 logic product terms assigned to macrocell 8 in block A. In the MACH435, since the signal is synchronous, the PT cluster size for macrocell 8 is 4, and it is steered to macrocell 8. The single XOR product term is also steered to macrocell 8, and is used as a logic PT. See the Cluster to Mcell (macrocell) and XOR to Mcell Assignment columns.

MACH355 and MACH 4xx

===========================================================================

< Block [A] >    Macrocell (MCell) Cluster Assignments

===========================================================================

+ Macrocell Number

| PT Requirements------ Logic  XOR+  +--- Macrocell PT Cluster Size

|      Sync/Async-------+   |     |  |    Cluster to Mcell Assignment

|   Node Fixed(*)----+  |   |     |  |      |   +- XOR PT Size

|        Sig Type-+  |  |   |     |  |      |   |  XOR to Mcell Assignment

|  Signal Name    |  |  |   |     |  |      |   |          |

_|_________________|__|__|___|_____|__|______|___|__________|______________

0|       t4_s[3]| IO| | S | 5      | 4 to [ 0]| 1 XOR to [ 0] as logic PT

1|              | ? | | S |    -   | 4 to [ 2]| 1 XOR to [ 2] as logic PT

2|      nsdstr_o|OUT| | S |17      | 4 to [ 2]| 1 XOR to [ 2] as logic PT

3|        ncmd_o| IO| | S | 2      | 4 to [ 2]| 1 XOR to [ 2] as logic PT

4|        nadl_o| IO| | S | 2      | 4 to [ 2]| 1 XOR free

5|              | ? | | S |    -   | 4 to [ 3]| 1 XOR free

6|        t4_s10|NOD|*| S | 1      | 4 to [ 4]| 1 XOR to [ 6] for 1 PT sig

7|              | ? | | S |    -   | 4 to [ 8]| 1 XOR to [ 8] as logic PT

8|       sdstr_o|OUT| | S |17      | 4 to [ 8]| 1 XOR to [ 8] as logic PT

9|       nactroe| IO| | A | 2 :+: 1| 2 to [ 8]| 1 XOR to [ 9]

10|              | ? | | S |    -   | 4 to [ 8]| 1 XOR to [ 8] as logic PT

11|              | ? | | S |    -   | 4 to [ 9]| 1 XOR free

12|        buswon|NOD|*| S | 1      | 4 free   | 1 XOR to [12] for 1 PT sig

13|              | ? | | S |    -   | 4 free   | 1 XOR free

14|              | ? | | S |    -   | 4 free   | 1 XOR free

15|              | ? | | S |    -   | 4 free   | 1 XOR free

---------------------------------------------------------------------------

Macrocell 8 provides 5 of the 17 product terms required by signal SDSTR_O, so the remaining 12 product terms must be steered from the adjacent macrocells. Macrocells 7 and 10 are unused, therefore an additional 10 product terms can be steered to macrocell 8.

Asynchronous I/O signal NACTROE is assigned to macrocell 9, and this changes the macrocell PT cluster size from 4 to 2. NACTROE requires 2 logic product terms that are XORed with another PT. The single XOR PT is obtained from macrocell 9, and because the 2 logic product terms in macrocell 9 were steered to fulfill the PT requirements of signal SDSTR_O in macrocell 8, NACTROE's logic requirements are satisfied by steering product terms from macrocell 11.

Node signals BUSWON and T4_S10 are fixed to macrocells (note '*' in Node Fixed column), and the I/O and Output signals in block A are also fixed (see Device Pin Out List). With the output switch matrix, the Placer can still move signals (except the fixed nodes) among macrocells to find macrocells with enough product terms to fulfill the signals' logic requirements.

Since the MACH 1xx and MACH 2xx parts do not have XOR gates in the macrocells, references to it will be removed in this section for these devices:

MACH210/MACH211/MACH220/MACH231

===========================================================================

< Block [A] >	Macrocell (MCell) Cluster Assignments

===========================================================================

+ Macrocell Number

| PT Requirements------ Logic        +--- Macrocell PT Cluster Size

|      Sync/Async-------+   |        |    Cluster to Mcell Assignment

|   Node Fixed(*)----+  |   |        |      |

|        Sig Type-+  |  |   |        |      |

|  Signal Name    |  |  |   |        |      |

_|_________________|__|__|___|________|______|_____________________________

0|              | ? | | S |    -   | 4 free   |

1|              | ? | | S |    -   | 4 free   |

2|              | ? | | S |    -   | 4 free   |

3|              | ? | | S |    -   | 4 free   |

4|              | ? | | S |    -   | 4 free   |

5|              | ? | | S |    -   | 4 free   |

6|          out5|OUT| | S | 2      | 4 to [ 6]|

7|       in_out5|Rin|*| S | 0      | 4 free   |

8|          out6|OUT| | S | 1      | 4 to [ 8]|

9|              | ? | | S |    -   | 4 free   |

10|         out12|OUT| | S | 1      | 4 to [10]|

11|              | ? | | S |    -   | 4 free   |

12|         out13|OUT| | S | 1      | 4 to [12]|

13|              | ? | | S |    -   | 4 free   |

14|              | ? | | S |    -   | 4 free   |

15|              | ? | | S |    -   | 4 free   |

---------------------------------------------------------------------------

Additional output enable (OE) PT banking information is provided in the following table for MACH 1xx parts. A MACH 1xx block has 4 OE PTs available for the macrocells in the block. In the case of the MACH110, 8 macrocells can choose between 2 OE PTs, and the other 8 macrocells can choose between the other 2 OE PTs. In the rightmost column of the MACH 2xx table above, we see that macrocells 0 to 7 can choose between OE PTs 0 and 1, and macrocells 8 to 15 can select between OE PTs 2 and 3.  All MACH 1xx macrocell output buffers can be enabled or disabled without using the output enable product terms.

MACH110/MACH111/MACH120/MACH130/MACH131

===========================================================================

< Block [A] >	Macrocell (MCell) Cluster Assignments

===========================================================================

+ Macrocell Number

| PT Requirements------ Logic        +--- Macrocell PT Cluster Size

|      Sync/Async-------+   |        |    Cluster to Mcell Assignment

|   Node Fixed(*)----+  |   |        |      |   +- OE PT key

|        Sig Type-+  |  |   |        |      |   |  Blk OE PTs

|  Signal Name    |  |  |   |        |      |   |          |

_|_________________|__|__|___|________|______|___|__________|______________

0|      io_am[1]| IO| | S | 6      | 4 to [ 0]|  2 => 40 |( 0)  1

1|      io_am[4]| IO| | S | 1      | 4 to [ 0]|  2 => 40 |( 0)  1

2|              | ? | | S |    -   | 4 to [ 1]| -  => -  |  0   1

3|              | ? | | S |    -   | 4 to [ 4]| -  => -  |  0   1

4|      io_am[2]| IO| | S | 9      | 4 to [ 4]|  1 => 30 |  0 ( 1)

5|      io_am[3]| IO| | S | 1      | 4 to [ 4]| -  => -  |  0   1

6|              | ? | | S |    -   | 4 to [ 5]| -  => -  |  0   1

7|              | ? | | S |    -   | 4 free   | -  => -  |  0   1

8|      io_am[0]| IO| | S | 1      | 4 to [ 8]|  0 => 90 |( 2)  3

9|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

10|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

11|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

12|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

13|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

14|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

15|              | ? | | S |    -   | 4 free   | -  => -  |  2   3

---------------------------------------------------------------------------

The MACH 1xx design example above has 5  logic equations with 3 unique OE PT equations.  Each of the 3 OE PT equations has been assigned a key value and are listed as follows:

0 = 90

1 = 30

2 = 40

Key values help identify valid locations for placing signals, as described below.

IO signal IO_AM[1] with 6 PTs has been assigned to macrocell number 0, and has the 3rd OE PT equation, 2, with the key value 40. The Placer indicates that block OE PT 0 has been allocated to this signal by putting the 0 in parentheses.

IO_AM[4] uses the same OE PT equation as IO_AM[1] (ie., key value 40), and the macrocell assignment table shows IO_AM[4] being assigned to node 1 (the second macrocell in the PAL block, because internal nodes are numbered starting at 0) with OE PT 0 also selected for this macrocell.

IO_AM[2] has a different OE PT equation, (ie., key value 30) and has been assigned to macrocell 4 and block OE PT 1.

IO_AM[3] requires only 1 PT, but since it does not require an OE PT equation (that is, the output signal is always enabled or disabled), it can be assigned to any macrocell capable of supporting 1 PT equations. The table above shows the Placer assigning IO_AM[3] to macrocell 5. Note that none of the block OE PTs has been selected for this signal.

IO_AM[0] requires only 1 PT, and if it did not have an OE PT equation, could have been assigned to macrocell 7. Since it has an OE PT equation with key value 90, the only valid locations for this signal are macrocells 8 through 15 since they provide a new set of block OE Pts (2 and 3). The table shows the Placer assigning IO_AM[0] to macrocell 8 and selecting block OE PT 2 for IO_AM[0].

#### Maximum PT Capacity

This table indicates the maximum number of product terms a signal can have with the current macrocell assignment.

Example

With the current macrocell assignment shown in the example in the preceding Macrocell Cluster Assignment section, T4_S[3] can support only 5 product terms. Signal NSDSTR_O in macrocell 2 can support up to 20 product terms because it has PT clusters from macrocells 1, 2, 3, and 4 steered to it, and the XOR product terms from these macrocells can also be used as extra logic product terms.

===========================================================================

< Block [A] >   Maximum PT Capacity

===========================================================================

+ Macrocell Number

| PT Requirements------ Logic  XOR+

|      Sync/Async-------+   |     |

|   Node Fixed(*)----+  |   |     |

|        Sig Type-+  |  |   |     |

|  Signal Name    |  |  |   |     |     Maximum PT Capacity

_|_________________|__|__|___|_____|_______________________________________

0|       t4_s[3]| IO| | S | 5      |=> can support up to [  5] logic PT(s)

1|              | ? | | S |    -   |=> [  0] PT capacity

2|      nsdstr_o|OUT| | S |17      |=> can support up to [ 20] logic PT(s)

3|        ncmd_o| IO| | S | 2      |=> can support up to [  5] logic PT(s)

4|        nadl_o| IO| | S | 2      |=> can support up to [  4] logic PT(s)

5|              | ? | | S |    -   |=> can support up to [  1] logic PT(s)

6|        t4_s10|NOD|*| S | 1      |=> can support up to [  1] logic PT(s)

7|              | ? | | S |    -   |=> [  0] PT capacity

8|       sdstr_o|OUT| | S |17      |=> can support up to [ 17] logic PT(s)

9|       nactroe| IO| | A | 2 :+: 1|=> can support up to [  5] logic PT(s)

10|              | ? | | S |    -   |=> can support up to [  4] logic PT(s)

11|              | ? | | S |    -   |=> can support up to [ 10] logic PT(s)

12|        buswon|NOD|*| S | 1      |=> can support up to [ 15] logic PT(s)

13|              | ? | | S |    -   |=> can support up to [ 19] logic PT(s)

14|              | ? | | S |    -   |=> can support up to [ 15] logic PT(s)

15|              | ? | | S |    -   |=> can support up to [ 10] logic PT(s)

---------------------------------------------------------------------------

Macrocell 1 is unusable because even though it is unused, all the PT clusters available to it have been steered to other macrocells. Its maximum PT capacity is therefore 0.

#### Node-Pin Assignments

An output switch matrix (OSM) lets a macrocell go to different I/O pins. The MACH355 and MACH 4xx OSM lets a macrocell go to 4 I/O pins.

The following is a sample macrocell-to-I/O pin table in the PRD file.

MACH435

===========================================================================

< Block [A] >    Node-Pin Assignments

===========================================================================

+ Macrocell Number

| Node Fixed(*)------+

|      Sig Type---+  |  to |  Block [A] IO Pin  |   Device Pin

|  Signal Name    |  | pin |      Numbers       |     Numbers

_|_________________|__|_____|____________________|________________________

0|       t4_s[3]| IO| | => |(  5)   6    7    0 |(  8)   9   10    3

1|              | ? | | => |   5    6    7    0 |   8    9   10    3

2|      nsdstr_o|OUT| | => |   6    7    0 (  1)|   9   10    3 (  4)

3|        ncmd_o| IO| | => |   6 (  7)   0    1 |   9 ( 10)   3    4

4|        nadl_o| IO| | => |   7 (  0)   1    2 |  10 (  3)   4    5

5|              | ? | | => |   7    0    1    2 |  10    3    4    5

6|        t4_s10|NOD|*| => |   0    1    2    3 |   3    4    5    6

7|              | ? | | => |   0    1    2    3 |   3    4    5    6

8|       sdstr_o|OUT| | => |   1    2    3 (  4)|   4    5    6 (  7)

9|       nactroe| IO| | => |   1 (  2)   3    4 |   4 (  5)   6    7

10|              | ? | | => |   2    3    4    5 |   5    6    7    8

11|              | ? | | => |   2    3    4    5 |   5    6    7    8

12|        buswon|NOD|*| => |   3    4    5    6 |   6    7    8    9

13|              | ? | | => |   3    4    5    6 |   6    7    8    9

14|              | ? | | => |   4    5    6    7 |   7    8    9   10

15|              | ? | | => |   4    5    6    7 |   7    8    9   10

--------------------------------------------------------------------------

Each macrocell can be routed to some but not all pins in the PAL block.  This table lists, for each macrocell, the available block pins and the corresponding physical device pins to which it can be routed.  The block pin and device pin to which it actually was routed are enclosed in parentheses.

The sample table above shows that output signal T4_S[3] in macrocell 0 was assigned to block A I/O pin 5 (the corresponding physical device pin number is 8).  Macrocell 0 could have been routed to either block pins 5, 6, 7 or 0 (device pins 8, 9, 10, or 3). Note that internal signals/nodes are not assigned to I/O pins.

MACH 1xx macrocells are directly connected to IO pins; therefore each node has only 1 destination pin and each pin has only one node source:

MACH 1xx

===========================================================================

< Block [A] >	Node-Pin Assignments

===========================================================================

+ Macrocell Number

| Node Fixed(*)------+

|      Sig Type---+  |  to |  Block [A] IO Pin  |   Device Pin

|  Signal Name    |  | pin |      Numbers       |     Numbers

_|_________________|__|_____|____________________|________________________

0|      io_am[1]| IO| | => |     (  0)          |     (  2)

1|      io_am[4]| IO| | => |     (  1)          |     (  3)

2|              | ? | | => |        2           |        4

3|              | ? | | => |        3           |        5

4|      io_am[2]| IO| | => |     (  4)          |     (  6)

5|      io_am[3]| IO| | => |     (  5)          |     (  7)

6|              | ? | | => |        6           |        8

7|              | ? | | => |        7           |        9

8|      io_am[0]| IO| | => |     (  8)          |     ( 14)

9|              | ? | | => |        9           |       15

10|              | ? | | => |       10           |       16

11|              | ? | | => |       11           |       17

12|              | ? | | => |       12           |       18

13|              | ? | | => |       13           |       19

14|              | ? | | => |       14           |       20

15|              | ? | | => |       15           |       21

---------------------------------------------------------------------------

For MACH 2xx parts, the odd numbered nodes are internal nodes only (ie., they cannot connect to IO pins).

Designers can specify input registers for MACH 2xx designs. An input register in a MACH 2xx will use one of the internal macrocell nodes. If a MACH210 design has an input pin INP_PIN paired with an input register IREG_NOD, then the Placer will assign INP_PIN to one of the IO pins and will then assign IREG_NOD to the internal node associated with that IO pin.

In the node-pin assignment table below, IREG_NOD (with registered input type Rin) has been assigned to internal node 1 (the second macrocell in PAL block A).

MACH 2xx

===========================================================================

< Block [A] >	Node-Pin Assignments

===========================================================================

+ Macrocell Number

| Node Fixed(*)------+

|      Sig Type---+  |  to |  Block [A] IO Pin  |   Device Pin

|  Signal Name    |  | pin |      Numbers       |     Numbers

_|_________________|__|_____|____________________|________________________

0|      nd_de[0]|NOD| | => |        0           |   2

1|      ireg_nod|Rin| | => |   (internal node)

2|     out_am[1]| IO| | => |     (  1)          |(  3)

3|      nd_am[1]|NOD| | => |	(internal node)

4|      nd_am[2]|NOD| | => |        2           |   4

5|              | ? | | => |	(internal node)

6|              | ? | | => |        3           |   5

7|              | ? | | => |	(internal node)

8|              | ? | | => |        4           |   6

9|              | ? | | => |	(internal node)

10|              | ? | | => |        5           |   7

11|              | ? | | => |	(internal node)

12|              | ? | | => |        6           |   8

13|              | ? | | => |	(internal node)

14|              | ? | | => |        7           |   9

15|              | ? | | => |	(internal node)

---------------------------------------------------------------------------

#### IO-to-Node Pin Mapping

This table shows what macrocells an I/O pin can connect to through the OSM; this is a complementary view to the Node-Pin Assignments table.  As in the Node-Pin Assignments table, actual mappings are indicated by placing parentheses around the node number to which the signal was actually routed.  If a signal is assigned to an I/O pin, then it can take its logic from 8 different nodes in the MACH355 and MACH 4xx devices.

In the table below, I/O signal NADL_O is fixed to I/O pin 0, and is connected to macrocell 4.

MACH 3xx/4xx

===========================================================================

< Block [A] >    IO-to-Node Pin Mapping

===========================================================================

+- Block IO Pin

|  Device Pin No.--------+

|    Pin Fixed(*)----+   |

|       Sig Type--+  |   |     |

|     Signal Name |  |   |     |  Node Destinations Via Output Matrix

_|_________________|__|___|_____|__________________________________________

0|        nadl_o| IO|*|  3| => |   0    1    2    3  ( 4)   5    6    7

1|      nsdstr_o|OUT|*|  4| => | ( 2)   3    4    5    6    7    8    9

2|       nactroe| IO|*|  5| => |   4    5    6    7    8  ( 9)  10   11

3|              | ? | |  6| => |   6    7    8    9   10   11   12   13

4|       sdstr_o|OUT|*|  7| => | ( 8)   9   10   11   12   13   14   15

5|       t4_s[3]| IO|*|  8| => |  10   11   12   13   14   15  ( 0)   1

6|              | ? | |  9| => |  12   13   14   15    0    1    2    3

7|        ncmd_o| IO|*| 10| => |  14   15    0    1    2  ( 3)   4    5

---------------------------------------------------------------------------

Block A I/O pins 3 and 6 are unused, therefore you can add 2 more signals requiring I/O pins to block A, but these signals can only be connected to the macrocells listed in the table for the I/O pins.

MACH 1xx macrocells are directly connected to IO pins; therefore each node has only 1 destination pin and each pin has only one node source:

MACH 1xx

===========================================================================

< Block [A] >	IO-to-Node Pin Mapping

===========================================================================

+- Block IO Pin

|  Device Pin No.--------+

|    Pin Fixed(*)----+   |

|       Sig Type--+  |   |     |

|     Signal Name |  |   |     |  Node Destinations Via Output Matrix

_|_________________|__|___|_____|___________________________________________

0|      io_am[1]| IO| |  2| => | ( 0)

1|      io_am[4]| IO| |  3| => | ( 1)

2|              | ? | |  4| => |   2

Continued...

...Continued

3|              | ? | |  5| => |   3

4|      io_am[2]| IO| |  6| => | ( 4)

5|      io_am[3]| IO| |  7| => | ( 5)

6|              | ? | |  8| => |   6

7|              | ? | |  9| => |   7

8|      io_am[0]| IO| | 14| => | ( 8)

9|              | ? | | 15| => |   9

10|              | ? | | 16| => |  10

11|              | ? | | 17| => |  11

12|              | ? | | 18| => |  12

13|              | ? | | 19| => |  13

14|              | ? | | 20| => |  14

15|              | ? | | 21| => |  15

---------------------------------------------------------------------------

Note that the type for the INP_PIN signal is IN2 in the following report.  The "IN2" designation means that INP_PIN is an input pin with a registered input in a MACH 2xx device.

MACH 2xx

===========================================================================

< Block [A] >	IO-to-Node Pin Mapping

===========================================================================

+- Block IO Pin

|  Device Pin No.--------+

|    Pin Fixed(*)----+   |

|       Sig Type--+  |   |     |

|     Signal Name |  |   |     |  Node Destinations Via Output Matrix

_|_________________|__|___|_____|___________________________________________

0|       inp_pin|in2| |  2| => |   0

1|     out_am[1]| IO| |  3| => | ( 2)

2|              | ? | |  4| => |   4

3|              | ? | |  5| => |   6

4|              | ? | |  6| => |   8

5|              | ? | |  7| => |  10

6|              | ? | |  8| => |  12

7|              | ? | |  9| => |  14

---------------------------------------------------------------------------

#### IO/Node and IO/Input Macrocell Pairing Table

This table shows Input, I/O or output signals that are paired to input registers or nodes. In the following example, the I/O signal NADL_O which is fixed to I/O pin 0 in block A (or device pin number 3) is not paired with an input register but is paired with the node signal RN_NADL_O.

===========================================================================

< Block [A] > IO/Node and IO/Input Macrocell Pairing Table

===========================================================================

+- Block IO Pin

|  Device Pin No.--------+

|    Pin Fixed(*)----+   |

|       Sig Type--+  |   |     |

|     Signal Name |  |   |     |  Input Macrocell and Node Pairs

_|_________________|__|___|_____|_________________________________________

0|        nadl_o| IO|*|  3| => | Input macrocell   [             -]

|              |   | |   |    | IO paired w/ node [     rn_nadl_o]

1|      nsdstr_o|OUT|*|  4| => | Input macrocell   [             -]

2|       nactroe| IO|*|  5| => | Input macrocell   [             -]

|              |   | |   |    | IO paired w/ node [    rn_nactroe]

3|              | ? | |  6| => | Input macrocell   [             -]

4|       sdstr_o|OUT|*|  7| => | Input macrocell   [             -]

5|       t4_s[3]| IO|*|  8| => | Input macrocell   [             -]

|              |   | |   |    | IO paired w/ node [    rn_t4_s[3]]

6|              | ? | |  9| => | Input macrocell   [             -]

7|        ncmd_o| IO|*| 10| => | Input macrocell   [             -]

|              |   | |   |    | IO paired w/ node [     rn_ncmd_o]

---------------------------------------------------------------------------

The IO/Node and IO/Input Macrocell Pairing Table for the MACH 2xx parts will show the link between the 2 signals INP_PIN and IREG_NOD. Since OUT_AM[1] does not have an input macrocell, it does not have an input macrocell entry.

===========================================================================

< Block [A] >	IO/Node and IO/Input Macrocell Pairing Table

===========================================================================

+- Block IO Pin

|  Device Pin No.--------+

|    Pin Fixed(*)----+   |

|       Sig Type--+  |   |     |

|     Signal Name |  |   |     |  Input Macrocell and Node Pairs

_|_________________|__|___|_____|__________________________________________

0|       inp_pin|in2| |  2| => | Input macrocell   [      ireg_nod]

1|     out_am[1]| IO| |  3| => | Input macrocell   [             -]

2|              | ? | |  4| => | Input macrocell   [             -]

3|              | ? | |  5| => | Input macrocell   [             -]

4|              | ? | |  6| => | Input macrocell   [             -]

5|              | ? | |  7| => | Input macrocell   [             -]

6|              | ? | |  8| => | Input macrocell   [             -]

7|              | ? | |  9| => | Input macrocell   [             -]

---------------------------------------------------------------------------

#### Input and Central switch matrix tables

These sections of the report contain information on utilization of the Input Switch Matrix (ISM) and the Central Switch Matrix (CSM).

#### Input Multiplexer (IMX) Assignments

Each block in the MACH355 and MACH 4xx devices has input multiplexers (IMX) that provide inputs into the central switch matrix. The MACH435 has 8 IMXs per block, where each IMX is physically located between 2 macrocells and is used to select any 3 of 4 signals to pass on to the central switch matrix: the 2 macrocell nodes, one I/O pin, and the corresponding input register associated with the I/O pin.

In MACH 3xx/4xx designs, the IMX value represents all feedback from the pin and its associated macrocells.  This value is reported for MACH 1xx/2xx designs even though MACH 1xx/2xx do not have an input multiplexer.

Example

In the MACH435 example below, each IMX in a block and the signals that go through the IMX are shown. In IMX 0, I/O signal UPB[1] is fixed to block D I/O pin 0, and feedback from it through the I/O pin path is required (note '*' in Signal Fixed to Pin/Mcell column). The product terms for UPB[1] are in macrocell 3 and connected to I/O pin 0 through the OSM; node feedback is not required from UPB[1].

Even though 4 signals are used in IMX 0, this is still a valid placement because only 2 signals have to be fed back through the IMX to the central switch matrix (see '*' in Feedback Required column).

===========================================================================

< Block [D] > Input Multiplexer (IMX) Assignments

===========================================================================

+----- IO pin/Input Register, or Macrocell

IMX No.    |    +---- Block IO Pin or Macrocell Number

|       |    |  MACHXL Node/    +-- Signal using the Pin or Macrocell

|       |    |  Pin Number      |      +- Signal Fixed (*) to Pin/Mcell

|       |    |   |  Sig Type    |      | +- Feedback Required

---|-------|----|---|---|----------|------|-|------------------------------

IMX  0 [IOpin  0 | 40| IO         upb[1]|*|*]

[RegIn  0 |154|Rin      bufreg[1]| |*] paired w/[        upb[1]]

[MCell  0 | 50| IO         upb[0]| | ]

[MCell  1 | 51| IO         upb[3]| | ]

IMX  1	[IOpin  1 | 39| IO         upb[2]| |*]

[RegIn  1 |155|Rin      bufreg[2]| |*] paired w/[        upb[2]]

[MCell  2 | 52| IO         upb[7]| | ]

[MCell  3 | 53| IO         upb[1]| | ]

IMX  2	[IOpin  2 | 38| IO         upb[5]| |*]

[RegIn  2 |156|Rin      bufreg[5]| |*] paired w/[        upb[5]]

[MCell  4 | 54| IO         upb[2]| | ]

[MCell  5 | 55|NOD      fmtregsel| |*]

IMX  3	[IOpin  3 | 37| IO         upb[4]| |*]

[RegIn  3 |157|Rin      bufreg[4]| |*] paired w/[        upb[4]]

[MCell  6 | 56| IO         upb[4]| | ]

[MCell  7 | 57|NOD txvcihdrlsbsel| |*]

IMX  4	[IOpin  4 | 36|INP       upadr[0]| |*]

[RegIn  4 |158|                 -| | ]

[MCell  8 | 58|NOD    txvcihdr[3]| |*]

[MCell  9 | 59|NOD    txvcihdr[7]| |*]

Continued...

...Continued

IMX  5	[IOpin  5 | 35| IO         upb[0]| |*]

[RegIn  5 |159|Rin      bufreg[0]| |*] paired w/[        upb[0]]

[MCell 10 | 60| IO         upb[5]| | ]

[MCell 11 | 61|NOD txvcihdrmsbsel| |*]

IMX  6	[IOpin  6 | 34| IO         upb[3]| |*]

[RegIn  6 |160|Rin      bufreg[3]| |*] paired w/[        upb[3]]

[MCell 12 | 62|                 -| | ]

[MCell 13 | 63|NOD        coment1| |*]

IMX  7	[IOpin  7 | 33| IO         upb[7]| |*]

[RegIn  7 |161|Rin      bufreg[7]| |*] paired w/[        upb[7]]

[MCell 14 | 64|NOD      bufregsel| |*]

[MCell 15 | 65|                 -| | ]

---------------------------------------------------------------------------

If all 4 signals going through an IMX have to be fed back, the MACH435 IMX architecture cannot support this and one of the signals must be moved to another macrocell or I/O pin to alleviate congestion in this IMX. If all signals are fixed to pins or nodes, or all IMXs in the block are up to their limits, then the Placer will generate the message "Input Multiplexer congestion in block [x]" and the designer will have to remove signals from the block to correct the problem.

#### Logic Array Fan-in

The Logic Array Fanin table shows the signal that uses each mux in a logic block.

For example, in the MACH435, each logic block has 33 muxes, each of which chooses one of 18 signals from the central switch matrix. The set of 18 signals per mux will overlap with other muxes in the same block, and may cause the design to be unroutable if some signals are fixed.

In the Logic Array Fan-In table, the signals using each of the 33 muxes and the current placements of these signals are shown, along with the relative MACHXL node and pin numbers in a logic block.  Ellipses (...) indicate unused muxes.

===========================================================================

< Block [H] > Logic Array Fan-in

===========================================================================

+- Central Switch Matrix No.

|          Signal       Source     MACHXL Node/Pin Numbers

--|--|-----------------------------|---------------------------------------

Mux00|      master_rs  IO Pin G  5 |   71

Mux01|         nempty  IO Pin E  5 |   50

Mux02|     rn_nactroe   Mcell A  9 |   11

Mux03|            ...         ...  |

Mux04|         intrst  IO Pin H  7 |   75

Mux05|        rn_s[0]   Mcell F  0 |   82

Mux06|    nucsfdbkrtn  IO Pin H  6 |   76

Mux07|     rn_t1_s[1]   Mcell H  6 |  120

Mux08|            ...         ...  |

Mux09|  rn_nuchbuserr   Mcell C  6 |   40

Mux10|     rn_t1_s[0]   Mcell D  4 |   54

Mux11|        rn_s[3]   Mcell F  6 |   88

Mux12|            ...         ...  |

Mux13|        en_xfer   Input Pin  |   41

Mux14|    rn_error[1]   Mcell H  2 |  116

Mux15|        rn_s[2]   Mcell F  2 |   84

Mux16|            ...         ...  |

Mux17|            ...         ...  |

Mux18|            ...         ...  |

Mux19|            ...         ...  |

Mux20|        rn_s[1]   Mcell F  8 |   90

Mux21|     nucds32rtn  IO Pin H  5 |   77

Mux22|     nova_cycle   Mcell G 10 |  108

Mux23|    rn_error[0]   Mcell H  0 |  114

Mux24|            ...         ...  |

Mux25|            ...         ...  |

Mux26|            ...         ...  |

Mux27|            ...         ...  |

Mux28|            ...         ...  |

Mux29|            ...         ...  |

Mux30|            ...         ...  |

Mux31|            ...         ...  |

Mux32|            ...         ...  |

---------------------------------------------------------------------------

Example

The table above shows macrocell node 5 in block G (G5) being selected by MUX00.  Suppose that G5 is one of the 18 inputs into mux 0, 20, and 27 of block H. If these muxes are used to select some other signals as array inputs into block H, then the signal assigned to G5 cannot be routed to block H. If the signal is not fixed to macrocell G5, then the Placer can try to move the signal to another macrocell in an attempt to look for muxes that are currently unused.

If the Placer has attempted all possible placements for unrouted signals, then the design is unfittable. The unrouted signal will be listed in the Logic Array Fan-In section, and will also be marked with the symbol '~' in the Signal Fan-Out table described earlier.

Example

The following design was 96% routed (but failed to route completely). The unrouted signal is B[4] and the block that it cannot route to is block D. The fan-out entry for B[4] to block D is preceded by a '~'.

* Routing Completion:  96% (252 of 260 signals routed)

==> Design unplaced and/or unrouted

Warning: {Unplaced and/or unrouted design.}

Errors : {No alternate placements for signals feeding the unrouted block [A].}

* Attempts: Place [    2144] Route [       2]

#$ Warnings or errors $#

===========================================================================

Signal Fanout Table

===========================================================================

+- Signal Number

|                +- Block Location ('+' for dedicated inputs)

|                |  + Sig Type

|  Signal Name   |  |      Fanout to Logic Blocks

---|--------------|-|---|--------------------------------------------------

1|a[1]          |A| IO|=> A . . . . . . . => Paired w/: [        an1[1]]

2|a[2]          |B| IO|=> . B . . . . . . => Paired w/: [        an1[2]]

3|a[3]          |C| IO|=> . . C . . . . . => Paired w/: [        an1[3]]

...

19|b[3]          |B|INP|=> A B C D E F G . => Paired w/: - none -

20|b[4]          |B|INP|=> A B C~D E F G . => Paired w/: - none -

Continued...

...Continued

21|b[5]          |B|INP|=> A B C D E . G . => Paired w/: - none -

...

===========================================================================

< Block [D] > Logic Array Fan-in

===========================================================================

*** [ 1] Unrouted Signals

[b[4]]

+- Central Switch Matrix No.

|          Signal       Source     MACHXL Node/Pin Numbers

--|--|-----------------------------|---------------------------------------

Mux00|           g[8]  IO Pin G  7 |   73

Mux01|            ...         ...  |

To improve routability, do one of the following:

r	Float node signals which will let the Placer assign these signals to macrocells which may have access to unused routing paths, consequently freeing routing paths for use by the unrouted signals

r	Reduce the amount of logic in block D, thereby releasing some routing resources for signal B[4]

r	Remove the logic in block D that required the B[4] signal, therefore removing B[4] from the list of signals to route to block D

The first option does not affect any fixed pin assignments because only nodes are being floated.

## Using Place and Route Data to Limit Placements

You can create a LIM file to limit the number of macrocells and logic array inputs that can be used in each PAL block in a design.  Instructions on creating LIM files are given in Appendix C.

Review the Place and Route Data file (Design.PRD) to determine if some blocks are highly utilized while others are sparsely utilized.  Then create a LIM file to limit the number of signals per block to some number less than the maximum supported by the block.

Example

Each MACH435 block can have up to 33 logic array inputs, as shown in the Placement Completion section of the PRD file below.

Start: Fri Oct 29 12:32:39 1993

End  : Fri Oct 29 12:32:39 1993    $$$ Elapsed time: 00:00:00

===========================================================================

Design [pr_test.pds]

* Placement Completion

+- Block                       +------- IO Pins Available

|    +- Macrocells Available   |    +-- IO Pins Used

|    |    +- Signals to Place  |    |             +----- Logic Array Inputs

|    |    |    +- Placed       |    |             |   +- Array Inputs Used

_|____|____|____|_______________|____|_____________|___|________________

A | 16 |  8 |  8 => 100% |     8 |  6 =>  75% |  33 | 18 =>  54%

B | 16 |  7 |  7 => 100% |     8 |  7 =>  87% |  33 | 18 =>  54%

C | 16 |  9 |  9 => 100% |     8 |  8 => 100% |  33 | 27 =>  81%

D | 16 |  8 |  8 => 100% |     8 |  7 =>  87% |  33 | 26 =>  78%

E | 16 |  5 |  5 => 100% |     8 |  8 => 100% |  33 | 16 =>  48%

F | 16 |  4 |  4 => 100% |     8 |  7 =>  87% |  33 | 22 =>  66%

G | 16 |  7 |  7 => 100% |     8 |  8 => 100% |  33 | 16 =>  48%

H | 16 |  5 |  5 => 100% |     8 |  8 => 100% |  33 | 17 =>  51%

---|----|----|------------|-------|------------|-----|------------------

The Router may take a long time to find a 100% routing solution if all 33 inputs into a block are used. With all 33 array inputs used, the Router cannot assign inputs that are blocked to unused array input muxes (because there are none), and will have to call the Placer to reassign signals to try different routing resources.

To increase the number of routing resources available for logic array inputs to such blocks, specify a MAX_FANIN value that is less than the maximum fanin (see Appendix C, "Creating a LIM File," for more details). Use the Place/Route Resource Usage Table in the PRD file to determine which blocks have a logic array input number near or at the maximum.

# Timing Analysis Report

The Timing Analysis report (Design.TAL) summarizes the number of propagation delays associated with each signal in the design.  (The actual time associated with a propagation delay is provided in the device data book.)  Signals incur an additional propagation delay for each feedback through the device's central switch matrix.

Delay values are only estimates and do not reflect timing differences between storage types.

r	Signals will be listed in descending order (higher to lower) with respect to delay.

r	This information is provided even if partitioning, placing, or routing  fails.

The MACH Fitter computes four distinct delay types: TSU, TCO, TPD, and TCR.  The value of each delay type represents the number of passes the corresponding signal makes through the switch matrix and combinatorial logic.  The total delay is calculated from one pin or register to another pin or register.

These delay types are defined in the following table.

Delay Types, Timing Analysis Report

| Type | Description |
| --- | --- |
| Tsu | Set-Up Time The number of passes through the switch matrix made by a signal between an input pin and the logic input of a clocked storage element. (TSU is 0 for input pairing.) |
| Tco | Clocked Output-to-Pin Time The number of passes through the central switch matrix made by a signal between the output of a clocked storage element and an I/O pin. (TCO is 0 for a register driving a pin directly.) |
| Tpd | Propagation Delay Time The number of passes through the central switch matrix made by a signal between an input pin and an output pin (calculated for combinatorial equations only). |
| Tcr | Clocked Output-to-Register Time The number of passes through the central switch matrix made by a signal between the output of one clocked storage device and the logic input of another. |

In the timing analysis report, the delay types are labeled TSU, TCO, TPD, and TCR.  The format of the report is:

TSU	TCO	TPD	TCR

# passes	# passes	# passes	# passes

Signal Name	Min. Max.	Min. Max.	Min. Max.	Min. Max.

Each signal is evaluated for each applicable delay type.  For each delay type, a minimum and a maximum value is calculated.

The following sections contain examples of each delay type.

| ã | Note: All timing metrics are calculated from the perspective of the register referenced in the timing report. |
| --- | --- |

## TSU

TSU represents the number of switch matrix passes between an input pin and a register setup before clock.

µ §

For example, the equation illustrated above would appear in the design file as follows:

PIN  ?  A2  COMB

NODE ?  B2  REG

...

EQUATIONS

B2 = A2

After fitting, the timing analysis report for signals B2 looks like this:

µ §

Note that TSU would have a value of zero for pins paired with an input register, because input pairing does not entail a pass through the central switch matrix.

## TCO

TCO represents the number of switch matrix passes between a clocked register and an output pin.

µ §

For example, the equation illustrated above would appear in the design file as follows:

PIN  ?  A2  COMB

NODE ?  B2  REG

PIN  ?  C2  COMB

...

EQUATIONS

B2 = A2

C2 = B2

After fitting, the timing analysis report for signals C2 looks like this:

µ §

TCO has a value of zero when a register drives a pin directly.

## TPD

TPD represents the number of switch matrix passes between an input pin and an output pin.

µ §

For example, the equation illustrated above would appear in the design file as follows:

PIN  ?  A2  COMB

PIN  ?  B2  COMB

...

EQUATIONS

B2 = A2

After fitting, the timing analysis report for signal B2 looks like this:

µ §

## TCR

TCR represents the number of switch matrix passes between a clocked register and the register it drives (before clock).

µ §

For example, the equation illustrated above would appear in the design file as follows:

PIN  ?  A2  COMB

NODE ?  B2  REG

PIN  ?  C2  REG

...

EQUATIONS

B2 = A2

C2 = B2

After fitting, the timing analysis report for signals C2 looks like this:

µ §

# Failure Reports

The following sections describe the reports that result from the following three conditions:

r	Failure to Partition

r	Failure to Place

r	Failure to Route

## Failure to Partition

On failure to partition the fitting report will include the following sections:

1.	Fitter Options

2.	Device Resource Checks

3.	Block Partitioning Summary.  The column labeled "Macrocells Used" contains the accurate count from the last partitioning attempt.  The "Macrocells Unusable" columns does not appear, as this information is meaningless in the case of a design that could not be partitioned successfully.

4.	Signal Summary.

Pin Signals
The location of a signal that was not placed (either partitioned or not partitioned), is indicated with a question mark (?).

Node Signals
The block number of a signal that was not partitioned is replaced with a question mark (?).  The location of a signal that was neither partitioned nor placed is indicated with a double question mark (??).  The location of a signal that was partitioned but not placed is indicated by the block number followed by a question mark (for example, D?).

5.	Tabular Information containing information on the last best partition encountered. Tabular Information will include signal names but will include only the preplacement and block assignment information available at the time of the partitioning failure.  All global clock placement available at the time of failure is listed in the individual "Clock Mux" section for each block.  Equation polarity is shown in the tables labeled "BLOCK x LOGIC MACROCELLS & INPUT REGISTERS."  Data fields for which data is not available are indicated with an ellipsis (...).

6.	Reason for failure to place an offending signal, listed by block. A typical message is "PRODUCT TERM DOES NOT FIT IN THE BLOCK.  Insufficient clusters.  It requires 3 but only 1 available."

7.	List of all unpartitioned signals.

8.	Percent completion of the partitioning process.

A sample of the Partitioning Failure Report is given below.

***************************

* PARTITIONING FAILURE REPORT*

************************

Signal 'BCA[1]' cannot be placed in any block partition

for the following reasons:

BLOCK A -

Block FANIN LIMIT ( 33 ) exceeded.

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK B -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK C -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK D -

Block FANIN LIMIT ( 33 ) exceeded.

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK E -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK F -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

Continued...

...Continued

BLOCK G -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

BLOCK H -

PRODUCT TERM does not fit in the block.

Insufficient clusters.  It requires 1 but only 0 available.

The following signals remain to be partitioned

BCA[0]           BCA[10]           BCA[15]            BCA[1]

BCA[5]            BCA[6]            BCB[0]           BCB[10]

BCB[15]            BCB[1]            BCB[5]            BCB[6]

BCC[0]           BCC[10]           BCC[15]            BCC[1]

BCC[5]            BCC[6]           PCA[10]           PCA[15]

PCA[5]           PCB[10]           PCB[15]            PCB[5]

PCC[10]           PCC[15]            PCC[5]        RN_PCA[10]

RN_PCA[15]         RN_PCA[5]        RN_PCB[10]        RN_PCB[15]

RN_PCB[5]        RN_PCC[10]        RN_PCC[15]         RN_PCC[5]

81 of 129

## Failure to Place

On failure to place the fitting report will include the following sections:

1.	Fitter Options

2.	Device Resource Checks

3.	Block Partitioning

4.	Signal Summary

5.	Tabular Information
Information on the best placement yet encountered.  Tabular Information will include signal names, placement information of pin and node numbers successfully assigned.  This report will include as much routing information (such as central switch matrix numbers) as possible.  Data fields for which data is not available are indicated with an ellipsis (...).

6.	Unplaced Signals
The signals that could not be placed and the blocks where placement failed.

7.	Failure Report
Reason for failure to place offending signals by block, as shown below.

*****************************

* PLACEMENT  FAILURE REPORT *

*****************************

Signal "OUT[1]" cannot be placed or routed in BLOCK-A for the

following reason:

<Pterm steering limitation> or

<Output steering limitation>

## Failure to Route

On failure to route the fitting report will include the following sections:

1.	Fitter Options

2.	Device Resource Checks

3.	Block Partitioning

4.	Signal Summary.

5.	Tabular Information containing information on the last best placement encountered. Tabular Information will include signal names, complete placement information of pin and node numbers. Report will include routing information on signals successfully routed, such as central switch matrix numbers. Information that is not available will be indicated in the table by a series of dots "...".

6.	The name of the offending signal. The signal that could not be routed and the block were routing failed.

7.	Reason for failure to route offending signal.

***************************

* ROUTING  FAILURE REPORT *

***************************

Signal "OUT[1]" cannot be placed or routed in BLOCK-A for the

following reason:

<Input Switch matrix limitation> or

<Central Switch matrix limitation>

. . .

For MACH465 devices, the "Clock/Input Pins" line is changed to "Clock Pins" because signals delivered to the clock pins of the MACH465 device can only be routed through the block clock mechanism.

If the failure was due to grouped or preplaced signals in the design file, the error message will be preceded by an introduction such as the following:

Grouped or preplaced signal 'C[4]' cannot be partitioned in specified block for the following reasons:

...followed by the specific reason(s) for the failure.